SLIPSTREAM 4

HARDWARE REFERENCE GUIDE

Version 3.3

29 July, 2026

© Copyright 1988, 1989, 1990, 1992 and 1993

# Table of Contents

µTable of Contents	3

The SS4 System	4

Introduction	4

Architecture	5

Video Controller	8

Video timing	8

Video modes	11

Interrupts	14

Memory	15

Memory timing	16

Clock generation	16

Peripheral IO	16

Pinout	18

Memory Timing	22

The Blitter	23

Introduction	23

The Blitter	24

Architecture	24

The Data Path	24

The Address Generator	27

The Sequencer	29

Memory Interface and Interrupts	33

Modes of Operation	35

Character Painting	37

Blitter Command Format	39

Blitter I/O Registers	42

The Digital Sound Processor	44

Architecture	44

Data Address Space	46

DSP Instruction Set	50

Compact Disk Controller	55

Synchronous Serial Interface	55

Block Decoder	56

DMA Interface	57

Control Interface	57

# The SS4 System

## Introduction

The SS4 System is designed to be a high-performance low-cost audio and video engine. It achieves this by utilising three processors; one is a general purpose processor - a 386SX, and the other two are highly specialised co-processors - the Blitter and the DSP.

The system is intended to be as simple as possible, while still providing the necessary performance levels; and to provide as much flexibility as the programmer may wish.

### This Document

This document is intended to be a useful guide to the assembly language programmer who wishes to drive the hardware facilities provided in an effective manner. It is not for novices.

This first section discusses the computer system as a whole, and is followed by individual sections discussing each sub-module. The best approach is to read it through first, then to approach it as a reference guide once the overall context of sections is understood.

## Architecture

The SS4 System computer electronics are largely contained within one massive custom logic chip, known as an ASIC (for Application Specific Integrated Circuit). The system memory and 386SX CPU lie outside the ASIC. This diagram summarises the architecture:

µ §

Sharing the main memory bus are four bus masters; the DSP, the Compact Disk Interface, the Blitter and the 386SX. Only one of the bus masters may own the bus at any one time, and a priority for the bus exists, giving the Compact Disk Controller highest priority, followed by the DSP, the Blitter and the CPU, in descending order.

The video controller controls the bus, and provides the memory timing signals for memory devices attached to the bus. It also requires memory cycles, and has effectively the highest priority on the bus. It will suspend bus master operations during video lines for brief periods to fetch video display data, and to refresh dynamic RAM. It controls the interface with the 386SX CPU.

The DSP is a simple, very high-speed processor for sound synthesis; operating at up to 24 MIPs. It has access to the main bus via a DMA controller which allows it to read and write bytes or words into main memory. These transfers occur in short bursts, and are under DSP program control. The DSP actually executes programs and stores data in its own private high speed memory.

The Compact Disk Read DMA channel allows the system to transfer CD-ROM read data into main memory without any software overhead. It  may transfer data directly, and it also contains a CD-ROM block decoder.

The Blitter is a graphics processor for fast screen updates and animation, acting as a hardware graphics sub-routine for the CPU or DSP. It will become bus master throughout a Blitter program operation, and may therefore own the bus for considerable periods. However, its priority over the CPU is not total, as it may be requested to give up the bus to the CPU when an interrupt occurs.

The 386SX CPU is the lowest priority bus master at the system level, but has complete control of the other hardware, and so the use of the bus is entirely under its program control.

### I/O Map

| Addr |  | Write Register |  | Read Register |
| --- | --- | --- | --- | --- |
| 00 | VID_INT | Video interrupt line | LP0 | Horizontal light-pen |
| 02 | V_PERIOD | Video vertical period | LP1 | Vertical light-pen |
| 04 | V_DIS_BEG | Video vertical display begin | TEST | Test register |
| 06 | V_BLK_END | Video vertical blanking end | ACKR | Interrupt read |
| 08 | H_COUNT | Video horizontal counter |  |  |
| 0A | V_DIS_END | Video vertical display end |  |  |
| 0C | V_COUNT | Video vertical counter | STAT | Video controller status |
| 0E | V_BLK_BEG | Video vertical blanking begin |  |  |
| 10 | SCROLL0 | Screen address 0 |  |  |
| 12 | SCROLL1 | Screen address 1 |  |  |
| 14 | V_SYNC | Video vertical sync |  |  |
| 16 | ACKW | Interrupt acknowledge |  |  |
| 18 | MODE | Video mode |  |  |
| 1A | BORDER | Border colour |  |  |
| 1C | H_PERIOD | Video horizontal period |  |  |
| 1E | MASK | Pixel mask |  |  |
| 20 | INDEX | Palette index |  |  |
| 22 | H_BLK_END | Video horizontal blanking end |  |  |
| 24 | H_FCH_BEG | Video horizontal fetch begin |  |  |
| 26 | MEM | Memory configuration |  |  |
| 28 | GPR | General purpose output |  |  |
| 2A | MODE2 | Auxiliary video mode |  |  |
| 2C | INT_DIS | Interrupt enable |  |  |
| 2E | H_DIS_BEG | Video horizontal display begin |  |  |
| 30 | H_FCH_END | Video horizontal fetch end |  |  |
| 32 | H_DIS_END | Video horizontal display end |  |  |
| 34 | H_BLK_BEG | Video horizontal blanking begin |  |  |
| 36 | H_SYNC | Video horizontal sync |  |  |
| 38 | H_VSYNC | Video horizontal vertical sync |  |  |
| 3A | BANK | Memory bank |  |  |
| 3C | CHRW | Chroma divider |  |  |
| 40 | BLITPC0 | Blitter program address 0 | BLITDST0 | Blitter destination address 0 |
| 42 | BLITPC1 | Blitter program address 1 | BLITDST1 | Blitter destination address 1 |
| 44 | BLITCMD | Blitter command | BLITSRC0 | Blitter source address 0 |
| 46 | BLITCON | Blitter control | BLITSRC1 | Blitter source address 1 |
| 48 |  |  | BLITIN | Blitter inner count |
| 4A | BLITROT0 | Blitter diagnostics | BLITOUT | Blitter outer count |
| 4C | BLITROT1 | Blitter diagnostics | BLITSTAT | Blitter status |
| 4E | BLITROT2 | Blitter diagnostics |  |  |
| 60 | CDCMD | CD command |  |  |
| 62 | CDADDR0 | CD DMA address 0 |  |  |
| 64 | CDADDR1 | CD DMA address 1 |  |  |
| 66 | CDCNT0 | CD DMA counter 0 |  |  |
| 68 | CDCNT1 | CD DMA counter 1 |  |  |
| 6A | CDHDR0 | CD block header 0 |  |  |
| 6C | CDHDR1 | CD block header 1 |  |  |
| 80-9F | JOYOUT | Joystick output | JOYIN | Joystick input |
| A0-BF | GPIO1 | General purpose I/O decode 1 | GPIO1 | General purpose I/O decode 1 |
| C0-DF | GPIO2 | General purpose I/O decode 2 | GPIO2 | General purpose I/O decode 2 |
| E0-FF | GPIO3 | General purpose I/O decode 3 | GPIO3 | General purpose I/O decode 3 |

The 386SX I/O space is internally decoded to eight bits, as shown above. In addition to this, three spare general purpose I/O decodes are provided - these may be used to provide an active low chip enables to external devices.

All internal I/O locations are on even boundaries, and word wide I/O reads and writes may be performed where appropriate. Do not perform byte wide I/O writes on word-wide write registers, and do not perform any I/O cycles to odd addresses.

All unmarked locations are reserved for future expansion or manufacturing test modes. Do not write to any of these locations or use them for external I/O.

### Memory Map

The system memory map contains four areas, DRAM bank 0, DRAM bank 1, ROM, and internal memory. Note that on power up ROM appears throughput the address map (see below).

µ §

# Video Controller

This document describes the video section of the Slipstream IV ASIC.

The video section includes the following functions:

·	Video timing generator

·	Interrupt handling

·	Video display generation

·	Memory configuration and timing

## Video timing

SS4 has a flexible video timing generator which can be programmed to suit different TV standards and monitors up to VGA 640 x 480.

The position of synchronization pulses, blanking, display area,  active video (when the ASIC is fetching data from memory) are programmed in clock cycles in the horizontal dimension and in line numbers in the vertical direction.

Video timing is broken into two parts. Horizontal timing is defined in terms of clock cycles and is determined by a number of eleven bit registers. Vertical timing is defined in terms of display lines and is determined by a number of ten bit registers. In the formula which follow times are given in microseconds and frequencies in megahertz unless otherwise stated.

### H_PERIOD	Horizontal period	1Ch	WO

The value written to this register determines the horizontal line length in clock cycles. The line length is one greater than the number written to this register. The formula for the required number is:

H_PERIOD = (line length * clock frequency) - 1;

Common line lengths are 64mS  PAL, 63.5mS NTSC and 31.78mS VGA. Common clock frequencies are 22.168MHz PAL (chroma * 5), 21.4773 MHz NTSC (chroma * 6) and 25.17MHz VGA.

### H_SYNC	Horizontal Sync	36h	WO

The value written to this register determines the width of the horizontal sync pulse. This pulse is output every line except during vertical synchronization when longer pulses are generated. The width of horizontal sync in clock cycles is given by the difference between the horizontal period register and the horizontal sync register. The formula for the required number is:

H_SYNC = H_PERIOD - (horizontal sync width * clock frequency)

Common horizontal sync pulse widths are 4.7mS PAL and NTSC and 3.81mS VGA.

### H_BLK_END	Horizontal Blanking End	22h	WO

This register determines when horizontal blanking ends. The blanking after the end of sync is referred to as the "back porch". The width of the back porch in clock cycles is specified by this register. Common back porch widths are 5.7mS (PAL), 4.80mS (NTSC) and 1.91mS (VGA).

### H_BLK_BEG	Horizontal Blanking Begin	34h	WO

This register determines where horizontal blanking begins. The blanking before the start of horizontal sync is referred to as the "front porch". The formula for the required number is:

H_BLK_BEG = H_PERIOD - ((hsync width + front porch width) * clock frequency)

Common front porch widths are 1.65mS (PAL), 1.40mS (NTSC) and 0.64mS (VGA). The video is also blanked for several lines around vertical sync. This blanking is defined by V_BLK_BEG and V_BLK_END.

### H_DIS_BEG	Horizontal Display Begin	2Eh	WO

This register specifies how soon video is generated after the trailing edge of horizontal sync. The interval is specified in clock cycles. If H_DIS_BEG is greater than H_BLK_END the ASIC outputs the border colour in between.

N.B. H_BLK_END would not normally be greater than H_DIS_BEG as this would blank out the pixels between H_DIS_BEG and H_BLK_BEG.

The value written to this register would normally be chosen to put the picture in the middle of the screen. The formula for a number to do this is:

H_DIS_BEG = (H_BLK_END + H_BLK_BEG - (active display width * clock frequency)) / 2

Common active display widths are 52mS (PAL & NTSC) and 25.42mS (VGA). In practice smaller display widths would be used to display a convenient number of whole pixels.

### H_DIS_END	Horizontal Display End	32h	WO

This register specifies where the display ends and hence determines the width of the video display in pixels. It should be programmed with the following number

H_DIS_END = H_DIS_BEG + (number of pixels * clocks per pixel)

If H_BLK_BEG is greater than H_DIS_END then the border colour will be output until blanking begins.

### H_FCH_BEG	Horizontal Fetch Begin	24h	WO

This register determines where video fetches first start on the line. This should be programmed such that the sixteen byte pixel buffer has just been filled when the display begins. In practice this means that the value in H_FCH_BEG is given by the value in H_DIS_BEG less a constant which depends on the display mode.

|  |  | Clocks per pixel |  |  |
| --- | --- | --- | --- | --- |
|  |  | four | two | one |
| Bits per pixel | four | 128 | 64 | 32 |
|  | eight | 64 | 32 | 16 |
|  | sixteen | 32 | 16 | n/a |

### H_FCH_END	Horizontal Fetch End	30h	WO

This register determines where video fetches end on the line. In principal this is the value in H_DIS_END minus the above constant. However H_FCH_BEG should be rounded up so that H_FCH_END - H_FCH_BEG is a multiple of the above constant.

### H_VSYNC	Horizontal Vertical Sync	38h	WO

Vertical Sync is identified as wider sync pulses occurring on a number of lines. The width of these pulses is determined by this register which should be programmed as follows:

H_VSYNC = H_PERIOD - (vertical sync width * clock frequency)

Television uses 59.3mS vertical sync pulse widths. VGA has separated syncs so program H_VSYNC to make vertical sync as wide as possible (H_VSYNC = 0).

### V_PERIOD	Vertical Period	02h	WO

This register specifies the number of video lines per field. Common values are 312 (50Hz PAL), 262 (60Hz NTSC) and 525 (60Hz VGA).

### V_SYNC	Vertical Sync	14h	WO

This register determines the number of lines on which vertical sync is generated. It should be programmed as follows:

V_SYNC = V_PERIOD - lines of vertical sync

Common values are 3 lines (PAL & NTSC) and 1 line (VGA).

### V_BLK_END	Vertical Blanking End	06h	WO

This register determines how many lines are blanked after vertical sync. Common values are 20 (PAL), 15 (NTSC) and 33 (VGA)

### V_BLK_BEG	Vertical Blanking Begin	0Eh	WO

This register determines how many lines are blanked before vertical sync. It should be programmed as follows:

V_BLK_BEG = V_SYNC - lines of blanking prior to vertical sync

Common values are 2 (PAL & NTSC) and 10 (VGA)

### V_DIS_BEG	Vertical Display Begin	04h	WO

This register determines the first line of active video. If this register is greater than V_BLK_END the lines in between show the border colour. The display may be active on all the lines between V_BLK_END and V_BLK_BEG but some lines will be lost because TVs are over scanned. The number of video lines can be chosen to suit the application. To position the active area in the middle of the screen this register should be programmed as follows:

V_DIS_BEG = (V_BLK_END + V_BLK_BEG - number of active lines) / 2

Common active lines are 256 (50Hz PAL), 220 (60Hz NTSC), 480 (VGA).

### V_DIS_END	Vertical Display End	0Ah	WO

This register determines the last line of active video. If this register is less than V_BLK_BEG the lines in between will show the border colour. To position the active area in the middle of the screen this register should be programmed as follows:

V_DIS_END = (V_BLK_END + V_BLK_BEG + number of active lines) / 2

### VID_INT	Video Interrupt	00h	WO

This register determines the video line on which a video interrupt is generated. This interrupt may be enabled or disabled through the INT register. The interrupt occurs when the video mechanism stops at the end of the display line. It may be used by the processor to change display modes or to perform beam synchronous animation. The register may be re-programmed within a field to provide several interrupts per field.

### LP0, LP1	Light-pen Registers	00h, 02h	RO

The ASIC has a light-pen/light-gun input. The first rising edge on this input every field transfers the horizontal count and vertical count in to registers LP0 and LP1 respectively. These values can be converted to the X,Y co-ordinates of the pixel pointed at.

Bit 1 of register STAT is set if a rising edge on the light-pen input occurred this field. This should be read just prior to vertical sync.

### H_COUNT, V_COUNT Video Counters	08h, 0Ch	WO

These registers allow the video counters to be written directly for test purposes.

Example values for video are given below. The video modes are:

·	PAL	320 x 256, eight bits per pixel

·	NTSC	320 x 220, eight bits per pixel

·	VGA	640 x 480, eight bits per pixel

|  | 50Hz PAL | 60Hz NTSC | VGA |
| --- | --- | --- | --- |
| Clock frequency | 22.17MHz | 21.48MHz | 25.17MHz |
| H_PERIOD | 1418 | 1363 | 799 |
| H_SYNC | 1314 | 1262 | 703 |
| H_BLK_END | 126 | 103 | 48 |
| H_BLK_BEG | 1271 | 1232 | 688 |
| H_DIS_BEG | 378 | 348 | 48 |
| H_DIS_END | 1018 | 988 | 688 |
| H_FCH_BEG | 346 | 316 | 32 |
| H_FCH_END | 986 | 956 | 672 |
| H_VSYNC | 103 | 89 | 0 |
| V_PERIOD | 312 | 262 | 525 |
| V_SYNC | 309 | 259 | 524 |
| V_BLK_END | 20 | 15 | 34 |
| V_BLK_BEG | 307 | 257 | 514 |
| V_DIS_BEG | 35 | 26 | 34 |
| V_DIS_END | 291 | 246 | 514 |

N.B. after programming these registers enable the video timing generator by setting the VIDEN bit in register MODE2

## Video modes

There are three colour resolutions available, four bits per pixel, eight bits per pixel and sixteen bit per pixel. In four and eight bit modes the pixel is a logical colour which indexes an eighteen bit physical colour stored in the palette. In sixteen bit mode the pixel is a physical colour in which bits 0 to 4 are blue, bits 5 to 10 are green and bits 11 to 15 are red. Because there are six bits of green but only five bits of blue and red. The least significant bits of blue and red output from the chip are always zero in sixteen bit mode.

The border colour is a sixteen bit register which is displayed as a sixteen bit pixel.

In eight bit mode the pixel addresses the whole 256 x 18 palette. In four bit mode the pixel addresses sixteen entries from the palette in which case the top four bits of the address are supplied from the index register.

Two variations are available in eight bit mode. In colour hold mode if the pixel takes the value zero then the colour of the previous pixel is displayed. This can be used to fill large areas of colour simply by setting the left most pixel.

In variable resolution mode the most significant pixel determines whether the pixel is displayed as one seven bit pixel or two three bit pixels. If the bit is clear the pixel is displayed as one seven bit pixel if the bit is set then bits 0 to 2 are displayed first followed by bits 4 to 6. In this case the two high resolution pixels address eight entries from the palette the top five bits of the address are supplied from the index register. Variable resolution mode is useful for displaying small regions of high resolution text amid a lower resolution, but more colourful, background. This mode is not available in one clock per pixel resolution.

In eight bit mode any of the bits can be sacrificed and used for other purposes. For instance a bit could be used to identify "hot spots" for collision detection alternatively bits could be used to encode image "depth" so that one image can move in front of or behind another. To sacrifice a bit the same bit in the mask register is set and that bit will be replaced from the corresponding bit in the index register.

There are three widths of pixel one clock, two clocks and four clocks. These correspond to dot clocks of around 24MHz, 12MHz and 6MHz. The first of these is intended for monitors, the second is for good looking text on televisions and the last is for games use on TVs. The highest dot clock may not be used with the sixteen bits per pixel display mode. Two other combinations, 24MHz 8bit & 12MHz 16bit may only be used if 32 bit DRAM is fitted.

If external hardware is fitted the ASIC can gen-lock to an external video source and mix local video (incrust) with external video on a pixel by pixel basis.

The memory map of the screen is not tied to the video display width but is defined independently. The base address of the screen can be anywhere in DRAM. The width of the screen memory is a power of two from 128 to 2048 bytes. The height of the screen is a power of two from 32K to 2M bytes. Video addresses on the same line wrap within the smaller boundary.

This arrangement allows the screen to be placed within a larger virtual screen and panned and scrolled within it.

The video mode is defined by the following registers

### MODE	Video mode register	18h	WO

| Bits 0,1 | determine the number of bits per pixel as follows |  |
| --- | --- | --- |
|  | 0 | four bits per pixel |
|  | 1 | eight bits per pixel |
|  | 2 | 16 bits per pixel |
| Bits 2..3 | determine the pixel width in clock cycles |  |
|  | 0 | four clock cycle |
|  | 1 | two clock cycles |
|  | 2 | one clock cycles |
| Bits 4..6 | determines the first break in the video address and hence the display width in bytes |  |
|  | 0 | address bit 7 = 128 bytes |
|  | 1 | address bit 8 = 256 bytes |
|  | 2 | address bit 9 = 512 bytes |
|  | 3 | address bit 10 = 1024 bytes |
|  | 4 | address bit 11 = 2048 bytes |
| Bits 7..9 | determines the second break in the video address and hence the display height in bytes |  |
|  | 0 | a[14] / 32K |
|  | 1 | a[15] / 64K |
|  | 2 | a[16] / 128K |
|  | 3 | a[17] / 256K |
|  | 4 | a[18] / 512K |
|  | 5 | a[19] / 1M |
|  | 6 | a[20] / 2M |
|  | 7 | a[21] / 4M |
| Bit 10 | Genlock - turns the sync outputs into inputs which can reset the horizontal and vertical timers for rapid locking to an external video source (needs extra hardware). |  |
| Bit 11 | Incrustation - bit 3 of four bit pixels, bit 7 of eight bit pixels and bit 5 of 16 bit pixels are routed to an output of the chip to switch between internal/external video (needs extra hardware). |  |
| Bit 12 | Border Incrustation - if set the incrust output is set during the border. |  |
| Bit 13 | Variable resolution mode |  |
| Bit 14 | Colour Hold mode |  |

### MASK	Pixel Mask Register	1Eh	WO
INDEX	Palette Index Register	20h	WO

For every bit set in the mask register the corresponding bit in the pixel is replaced by the bit from the index register. The upper bits in the index register form the high part of the palette address for four bit pixels.

### BORDER	Border Colour Register	1Ah	WO

This 16 bit register defines the border colour. The colour is displayed in the same way as 16 bit pixels, bits 0 to 4 are blue, bits 5 to 10 are green and bits 11 to 15 are red.

### SCROLL0	Screen Address Register 0	10h	WO
SCROLL1	Screen Address Register 1	12h	WO

These two registers define the 24 bit base address of the screen in memory. This is the address of the top left pixel on the screen. The bottom sixteen bits of the address are written to SCROLL0 the top eight bits to SCROLL1. The screen must be on a word boundary if 16 bit DRAM is fitted and on a long-word boundary if 32 bit DRAM is fitted.

If SCROLL1 is written during active video then the next line of pixels will be fetched from that address. This allows split screens to be implemented.

Video lines have to start on a 32-bit boundary when 32-bit RAM is used, which may appear to be a problem. There are two possible solutions. When 16-bit pixels are used, this corresponds to a two pixel boundary. As game screens are usually double-buffered, if the second buffer is maintained at a one pixel offset relative to the first, then single pixel scrolling can be achieved. Alternatively, and this may be a far more attractive solution; if the display is over-scanned, then the screen start position can be modified using the H_DIS_BEG register in increments of one clock cycle. As the left and right hand edges of the video line are not visible, it does not matter that they move around by small amounts. Therefore program the screen start position down to 32-bit resolution in the SCROLL registers, and use H_DIS_BEG to program the remainder. This will allow single pixel scrolling at any resolution with a single display buffer.

### MODE2	Auxiliary Video Mode Register	2Ah	WO

This register provides additional control over video and various test logic.

| Bit 0 | OPEN | Enables the light pen registers so that the horizontal and vertical counters can be read. Mainly useful for reading the vertical count so that the time of arrival of analogue input interrupts can be determined. |  |
| --- | --- | --- | --- |
| Bit 1 | TLD | Test purposes only, write zero. |  |
| Bit 2 | VIDEN | Enables the video timer. |  |
| Bit 3,4 | REFRATE | Determines the refresh frequency. |  |
|  |  | 1 | clock/128 |
|  |  | 2 | clock/256 |
|  |  | 3 | clock/512 |
|  |  | Most DRAMs require a refresh frequency of 64kHz or above. The refresh controller waits until eight or more refresh cycles are required then requests the bus and does the required number of CAS before RAS cycles. |  |
| Bit 5 | TMODE | Test purposes only, write zero. |  |
| Bit 6 | MODEBUF | When set the video mode (MODE) is double buffered and can only change during blanking. Set this bit for clean mode changes in split screen operation. |  |
| Bit 7 | VPOL | Inverts the polarity of vertical sync. |  |
| Bit 8 | HPOL | Inverts the polarity of horizontal sync. |  |
| Bit 9 | NAND | Test purposes only, write zero. |  |

### The Palette		F10000 to F103FF

The palette is a 256 x 18 RAM. Each entry contains six bits of red green and blue. Each entry extends across two words. The blue and green bits appear in the low word. The red bits appear in the high word. Bits 2-7 of the low word are blue, bits 10-15 of the low word are green, and bits 2-7 of the high word.

To write to an entry in the palette you must first write the red bits to the high word, then the green and blue bits to the low word.

You should only write to the palette during border or blanking or speckles will appear on the video.

## Interrupts

The system supports six interrupt sources: video, three analogue interrupts, CD block decoder and DSP. The video, CD block decoder and DSP interrupts are described elsewhere.

The analogue interrupts allow simple analogue to digital converters to be implemented. In principal a mono-stable is implemented from a diode, a capacitor and a potentiometer. The capacitor is discharged by vertical sync and begins charging at a rate dependent on the potentiometer setting. When the voltage on the capacitor reaches the threshold of the input to the ASIC an interrupt is generated. The processor can then read the vertical counter to get a measure of how quickly the capacitor charged and hence the potentiometer setting.

All six interrupts can be independently enabled or disabled through the INT register.

### INT	Interrupt enable register	2Ch	WO

| Bit 0 | Enables the video interrupt |
| --- | --- |
| Bit 1 | Enables analogue interrupt number 1 |
| Bit 2 | Enables analogue interrupt number 2 |
| Bit 3 | Enables analogue interrupt number 3 |
| Bit 4 | Enables CD block decoder interrupt |
| Bit 5 | Enables DSP interrupt |

Each interrupt has its own vector in the range A0 to A5. A0 corresponds to video, A1 to analogue interrupt 1 and so on. Interrupts with a lower vector will generate an interrupt before those with a higher vector. The processors interrupt acknowledge cycle automatically clears the interrupt.

### ACKR Interrupt Read Register	06h	RO

This register lends some flexibility to the rigidly prioritised vectored interrupt scheme. The bottom six bits of this register reflect all pending interrupts. The correspondence between bits and interrupts is as before.

### ACKW Interrupt Acknowledge Write Register	16h	WO

Writing a one to any of the bottom six bits clears the corresponding interrupt.

## Memory

The ASIC decodes the 16Mbyte address range of the 386 into the following memory map:

µ §

The 64k of internal memory comprises of palette RAM, Blitter Registers and DSP

8000		DSP

0800		Reserved

0400		Blitter Registers

0000		Palette

The DRAM may be either 16 bits or 32 bits wide. This is determined by the ASIC during reset but does not directly affect the 386. Instead it allows the video to operate more quickly leaving more bandwidth available to other processors. Certain display and blitter modes are only possible with 32 bit memory. Two banks of DRAM may be attached. If small amounts of DRAM are attached then they will be repeated throughout the spaces shown above.

ROM is always 16 bits wide. Following reset the one megabyte window containing ROM and internal memory is repeated throughout the 16Mbyte address range. This allows for a variety of processors to boot with the ASIC. The memory map above is adopted the first time register MEM is written.

The ASIC performs page mode cycles on the DRAM wherever possible. These are quicker than normal cycles and occur if successive reads/writes are within the same page. The ASIC need to know the number of columns in the DRAM this is programmed in register MEM.

### MEM	Memory type	26h WO

Bits 0,1	Determine the numbers of columns in the DRAM

0...256

1...512

2...1024

3...2048

Common DRAM types and the number of columns are shown

DRAM type	Columns

64k x 16	256

256k x 4	512

256k x 16	512

512k x 8	512

1M x 4	1024

## Memory timing

There are seven types of transfer controlled by the memory controller

| Normal DRAM cycle | four clocks |
| --- | --- |
| Page mode DRAM cycle | two clocks |
| ROM cycles | six clocks |
| Internal memory | two clocks |
| External IO | six clocks |
| Interrupt acknowledge | two clocks |
| Internal IO | two clocks |

The 386 will cycle in one clock more than the actual transfer. Internal bus masters can cycle in the transfer time.

## Clock generation

The ASIC has a crystal oscillator for a crystal which is the 2X (two times speed) clock for the 386 and is a multiple of the television colour sub-carrier. This crystal clock is buffered and output to the 386. The same clock is put through a divide by two and this is output as the main system clock. This clock is input to the ASIC as a whole through another pin. The reason for outputting then inputting the clock is so that the relative skew between the 386 2X clock and the main system clock can be adjusted one way or the other by adding small delays to either path.

The crystal frequency also goes through a programmable divider which can divide by a number between 1 and 15 and produce an output wave form with an even mark-space ratio. This will be used as the television colour sub-carrier.

### CHR	Chroma Divider Register	3Ch	WO

This four bit register defines the ratio of the television colour sub-carrier (chroma) to the 2X crystal frequency. It should be programmed as follows:

CHR = (2X crystal frequency / chroma frequency) - 1;

## Peripheral IO

The ASIC produces five IO decodes for external peripherals and one general purpose output. The IO decodes are

| JOYIN | 80-9F | Active for IO reads only. Intended for reading joystick switches. |
| --- | --- | --- |
| JOYOUT | 80-9F | Active for IO writes only. Intended for driving joystick switch matrix. |
| GPIO1 | A0-BF | General purpose IO decode asserted for reads and writes. |
| GPIO2 | C0-DF | As GPIO1 |
| GPIO3 | E0-FF | As GPIO1 |

### GPOUT	General Purpose Output Register	28h WO

Bit 0	This bit is drives the general purpose output.

### STAT	Status Register	0Ch RO

| Bit 0 | PAL | This bit is used to read the PAL/NTSC input to the ASIC. this is read during reset and cannot be changed. By convention if set the video timing should be set up for PAL, if clear for NTSC. |
| --- | --- | --- |
| Bit 1 | LPSET | Indicates that there has been a light-pen input in the current field. Set by the light-pen cleared by vertical sync. |

### "Peephole" Operation

The ASIC can be put into a mode during reset after which it only responds to two word wide IO locations and 64k memory locations. The actual address of these locations is determined by a chip select input so can be determined externally. This mode allows the ASIC to occupy small gaps in the IO and memory map of the PC.

In order to address all the IO within the ASIC the regular IO address of the required register is first written to the lower word (a[1] low) then that register can be read or written at the upper word (a[1] high).

In order to address all the memory inside and outside the ASIC the 64k window can be moved to any 64k boundary in the 16M address space formerly decoded by the ASIC by writing to the bank register.

### BANK	Memory Bank Register	3Ah WO

This eight bit register provides the most significant eight bits (A[16..23]) when addressing memory in peephole mode. So for example if you want to read the palette, formerly at 0F10000h, you first write 0F1h to the bank register and then read the palette at the bottom of the 64k peephole location (determined externally by chip select).

### Analogue Inputs

The chip supports three analogue inputs, and a light-pen type input.

The analogue voltages are measured by comparing them to an externally generated voltage ramp which is reset every vertical video synchronization pulse. When the analogue voltage equals the ramp voltage an interrupt is generated, one interrupt being generated in every video frame for each of the analogue inputs. When one of these interrupts occurs, the CPU may read the video vertical counter, and from this estimate the analogue voltage being measured. The interrupt mechanism is described in greater detail in the Video Controller section below.

A light-pen, or other beam detecting device such as a gun, may also be connected. This function is part of the Video Controller described below.

## Pinout

The following list gives the ASIC pin names and types.

| 1 | VSS | POWER | 71 | INTR | B2 | 141 | MA_12 | B16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | VDD | POWER | 72 | MIO | IBUF | 142 | VDD | POWER |
| 3 | R_0 | B4 | 73 | VDD | POWER | 143 | MA_13 | B4 |
| 4 | R_1 | B4 | 74 | ADSL | IBUF | 144 | MA_14 | B4 |
| 5 | R_2 | B4 | 75 | DC | IBUF | 145 | MA_15 | B4 |
| 6 | R_3 | B4 | 76 | HLDA | IBUF | 146 | VDD | POWER |
| 7 | R_4 | B4 | 77 | BHEL | IBUF | 147 | MA_16 | B4 |
| 8 | R_5 | B4 | 78 | BLEL | IBUF | 148 | MA_17 | B4 |
| 9 | G_0 | B4 | 79 | PCLK | B4 | 149 | MA_18 | B4 |
| 10 | VDD | POWER | 80 | RDY | B4 | 150 | MA_19 | B4 |
| 11 | VSS | POWER | 81 | HOLD | B4 | 151 | VDD | POWER |
| 12 | G_1 | B4 | 82 | CSL | IBUF | 152 | MA_20 | B4 |
| 13 | G_2 | B4 | 83 | VSS | POWER | 153 | MA_21 | B4 |
| 14 | G_3 | B4 | 84 | MD_0 | BD4T | 154 | MA_22 | B4 |
| 15 | G_4 | B4 | 85 | MD_1 | BD4T | 155 | MA_23 | B4 |
| 16 | G_5 | B4 | 86 | MD_2 | BD4T | 156 | WEL_0 | B16 |
| 17 | VSS | POWER | 87 | MD_3 | BD4T | 157 | VDD | POWER |
| 18 | B_0 | B4 | 88 | MD_4 | BD4T | 158 | WEL_1 | B16 |
| 19 | B_1 | B4 | 89 | MD_5 | BD4T | 159 | WEL_2 | B16 |
| 20 | B_2 | B4 | 90 | VSS | POWER | 160 | WEL_3 | B16 |
| 21 | VDD | POWER | 91 | MD_6 | BD4T | 161 | VDD | POWER |
| 22 | B_3 | B4 | 92 | MD_7 | BD4T | 162 | VSS | POWER |
| 23 | B_4 | B4 | 93 | MD_8 | BD4T | 163 | RASL_0 | B16 |
| 24 | B_5 | B4 | 94 | VDD | POWER | 164 | VSS | POWER |
| 25 | VSS | POWER | 95 | MD_9 | BD4T | 165 | RASL_1 | B16 |
| 26 | HD_0 | BD4T | 96 | MD_10 | BD4T | 166 | CASL_0 | B16 |
| 27 | HD_1 | BD4T | 97 | VSS | POWER | 167 | VSS | POWER |
| 28 | HD_2 | BD4T | 98 | MD_11 | BD4T | 168 | VDD | POWER |
| 29 | HD_3 | BD4T | 99 | MD_12 | BD4T | 169 | CASL_1 | B16 |
| 30 | HD_4 | BD4T | 100 | MD_13 | BD4T | 170 | VDD | POWER |
| 31 | VSS | POWER | 101 | MD_14 | BD4T | 171 | ROMCSL | B16 |
| 32 | HD_5 | BD4T | 102 | MD_15 | BD4T | 172 | OEL | B16 |
| 33 | HD_6 | BD4T | 103 | MD_16 | BD4T | 173 | HSYNCL | BD4T |
| 34 | HD_7 | BD4T | 104 | VDD | POWER | 174 | VSYNCL | BD4T |
| 35 | HD_8 | BD4T | 105 | MD_17 | BD4T | 175 | TEST | IBUF |
| 36 | HD_9 | BD4T | 106 | MD_18 | BD4T | 176 | RESETIL | IBUF |
| 37 | HD_10 | BD4T | 107 | VSS | POWER | 177 | VDD | POWER |
| 38 | VSS | POWER | 108 | MD_19 | BD4T | 178 | RESETO | B4 |
| 39 | HD_11 | BD4T | 109 | MD_20 | BD4T | 179 | XTALI | OSC4CI |
| 40 | HD_12 | BD4T | 110 | MD_21 | BD4T | 180 | XTALO | OSC4CO |
| 41 | HD_13 | BD4T | 111 | MD_22 | BD4T | 181 | AI_0 | IBUF |
| 42 | VDD | POWER | 112 | MD_23 | BD4T | 182 | AI_1 | IBUF |
| 43 | HD_14 | BD4T | 113 | MD_24 | BD4T | 183 | AI_2 | IBUF |
| 44 | HD_15 | BD4T | 114 | VDD | POWER | 184 | AISEL | B8 |
| 45 | HA_1 | IBUF | 115 | VSS | POWER | 185 | LP | IBUF |
| 46 | HA_2 | IBUF | 116 | MD_25 | BD4T | 186 | CLKOUT | B4 |
| 47 | HA_3 | IBUF | 117 | VDD | POWER | 187 | VSS | POWER |
| 48 | HA_4 | IBUF | 118 | MD_26 | BD4T | 188 | CLKIN | IBUF |
| 49 | HA_5 | IBUF | 119 | MD_27 | BD4T | 189 | SAEN | B4 |
| 50 | HA_6 | IBUF | 120 | VSS | POWER | 190 | CHROMA | B4 |
| 51 | HA_7 | IBUF | 121 | MD_28 | BD4T | 191 | VDD | POWER |
| 52 | HA_8 | IBUF | 122 | MD_29 | BD4T | 192 | INC | B4 |
| 53 | HA_9 | IBUF | 123 | MD_30 | BD4T | 193 | GPIOL_1 | B8 |
| 54 | HA_10 | IBUF | 124 | MD_31 | BD4T | 194 | GPIOL_2 | B8 |
| 55 | HA_11 | IBUF | 125 | VDD | POWER | 195 | GPIOL_3 | B8 |
| 56 | HA_12 | IBUF | 126 | VSS | POWER | 196 | JOYOUTL | B8 |
| 57 | HA_13 | IBUF | 127 | MA_0 | BD4T | 197 | JOYINL | B4 |
| 58 | HA_14 | IBUF | 128 | MA_1 | BD16T | 198 | VDD | POWER |
| 59 | HA_15 | IBUF | 129 | MA_2 | BD16T | 199 | SCLKB | IBUF |
| 60 | VSS | POWER | 130 | MA_3 | BD16T | 200 | SDATB | IBUF |
| 61 | HA_16 | IBUF | 131 | MA_4 | BD16T | 201 | VSS | POWER |
| 62 | HA_17 | IBUF | 132 | MA_5 | B16 | 202 | SERRB | IBUF |
| 63 | VSS | POWER | 133 | MA_6 | B16 | 203 | SWRDB | IBUF |
| 64 | HA_18 | IBUF | 134 | MA_7 | B16 | 204 | SDATIA | IBUF |
| 65 | HA_19 | IBUF | 135 | VSS | POWER | 205 | SDATOA | B4 |
| 66 | HA_20 | IBUF | 136 | MA_8 | B16 | 206 | SCLKA | BD4T |
| 67 | HA_21 | IBUF | 137 | MA_9 | B16 | 207 | SWRDA | BD4T |
| 68 | HA_22 | IBUF | 138 | VDD | POWER | 208 | VSS | POWER |
| 69 | HA_23 | IBUF | 139 | MA_10 | B16 |  |  |  |
| 70 | RDL | IBUF | 140 | MA_11 | B16 |  |  |  |

Key to the pin types

| B2,B4,B8,B16 | Outputs with 2mA, 4mA, 8mA, 16mA drive |
| --- | --- |
| BD4T, BD16T | Bi-directional pins with 4mA, 8mA drive TTL threshold inputs |
| IBUF | TTL level input buffer |
| OSCI, OSCO | Crystal Oscillator input and output |

### Pin Descriptions

| R[0..5] | red, green and blue outputs |
| --- | --- |
| G[0..5] | Intended to drive a 5k R/2R ladder. |
| B[0..5] |  |
| HD[0..15] | 386 data bus |
| HA[1..23] | 386 address bus |
| RDL | 386 read/write output |
| INTR | 386 Interrupt input |
| MIO | 386 memory/IO output |
| ADSL | 386 address status output, initiates all transfers. |
| DC | 386 data/control output |
| HLDA | 386 bus hold acknowledge output |
| BHEL,BLEL | 386 high byte, low byte enables |
| PCLK | 386 2X clock (output from ASIC) |
| RDY | 386 READY input terminates all transfers. |
| HOLD | 386 bus hold input |
| CSL | ASIC chip select. The ASIC will only respond to the 386 if this is pulled low. If high it tri-states RDY and HD[0..15]. This input can be used to define the peephole address or to disable the ASIC if the console is expanded with a turbo ASIC. |
| MD[0..31] | Memory data bus. Connects to ROM, DRAM and external IO data pins. Sixteen bit memory and IO connect to MD[0..15]. |
| MA[0..22] | Memory address bus. Connects to ROM, DRAM and external IO address pins. MA[1..12] are multiplexed if DRAM is being addressed. If 32 bit DRAM is attached the address inputs should be connected to MA[2] upwards. If 16 bit DRAM is attached the address inputs should be connected to MA[1] upwards. MA[0..2] are sensed during reset. They should be pulled up or down with 10k resistors to configure the ASIC as required. MA[0] notionally determines whether the video is PAL or NTSC. The sense is reflected in the status register but has no other effect on the hardware. MA[1] should be pulled high for operation with 32 bit DRAM, low for 16 bit DRAM. MA[2] should be pulled high for "peephole" operation low for normal operation. |
| WEL[0..3] | Memory write strobes. WEL[0] relates to MD[0..7], WEL[1] to MD[8..15], WEL[2] to MD[16..23] and WEL[3] to MD[24..31]. If 16 bit DRAM chips are fitted the types with two write strobes should be used rather than the types with two CAS inputs. |
| RASL[0..1] | Row address strobes for two banks of DRAM. Bank 0 is from 0 to 7FFFFFh (8Mbytes). Bank 1 is from 800000h to 0EFFFFFh (7Mbytes). |
| CASL[0..1] | Column address strobes for two banks of DRAM. |
| ROMCSL | ROM chip select, active low. |
| OEL | Memory output enable. |
| HSYNCL VSYNCL | Horizontal and vertical sync outputs. May be used as inputs if gen-lock bit is set. |
| TEST | Test input, should be grounded. |
| RESETIL | Active low reset input. |
| RESETO | Active high reset output. This output is synchronised so that the 386 internal clock has the same phase as CLKOUT (the main ASIC clock). |
| XTALI XTALO | Crystal oscillator. The crystal provides the 2X clock to the 386 and should also be a multiple of the colour sub-carrier frequency 4.43361875MHz or 3.579545MHz |
| AI[0..2] | Analogue interrupt inputs. A falling edge on these inputs can generate a 386 interrupt. |
| AISEL | General purpose output. |
| LP | Light-pen input. |
| CLKOUT CLKIN | Main ASIC clock output and input. Should be wired together. Delay may be inserted to solve any clock skew problems with the 386. |
| SAEN | Screen address enable. Identifies when the ASIC is performing a video access. |
| CHROMA | Colour sub-carrier output |
| INC | Incrust output. |
| GPIOL[1..3] | Active low general purpose IO decodes. |
| JOYOUTL | Active low joystick output decode. |
| JOYOUTL | Active low joystick input decode. |
| SCLKB | CD Block decoder synchronous serial clock input - connect to the clock input from the CD interface, or external block decoder. |
| SDATB | CD Block decoder synchronous serial data input - connect to the data input from the CD interface, or the external block decoder. |
| SERRB | CD Block decoder error input - connect to the error (or data unreliable) input from the CD interface, or tie low if not available. |
| SWRDB | CD Block decoder synchronous serial word select input - connect to the word select input from the CD interface, or to the external block decoder. |
| SDATIA | Audio synchronous serial data input - usually audio data in from CD. |
| SDATOA | Audio synchronous serial data output - usually audio data out to DAC |
| SCLKA | Audio synchronous serial data input/output - either connect to clock input from CD and to the DAC, so that this pin is an input; or connect just to the DAC, and enable this pin as an output and set up the internal audio time-base. |
| SWRDA | Audio synchronous serial word select input/output - connect as SCLKA to the appropriate word selects. |

## Memory Timing

The memory interface is synchronous so all signals originate from either the rising or falling edge of CLKIN. The following diagram is intended to show the relationship between

### RAS cycle followed by page mode cycle

____      ____      ____      ____      ____      ____      __ 
clk  __/    \____/    \____/    \____/    \____/    \____/    \____/  
     __________________                                      
ras  ____/             \______________________________________________
     ____ ________ _________ ___________________ ___________________ _
xma  ____X________X___row___X________col________X________col________X_
     ____________________________                ____                _
cas                              \______________/    \______________/

write____ ______________________________________ ___________________ _
data ____X______________________________________X___________________X_
     ________                               _________           ______
wel          \_____________________________/         \_________/

read                                         ____                ____
data ---------------------------------------X____X--------------X____X

### ROM cycle

____      ____      ____      ____      ____      ____      ___
clk  __/    \____/    \____/    \____/    \____/    \____/    \____/  
     ____ ___________________________________________________________ _
xma  ____X___________________________________________________________X_
     ____                                                             _
romcsl   \___________________________________________________________/

read                                                             ____
data -----------------------------------------------------------X____X

### CAS before RAS refresh cycles

____      ____      ____      ____      ____      ____      __ 
clk  __/    \____/    \____/    \____/    \____/    \____/    \____/  
     __________________                          __________________
ras  ____/             \________________________/                  \__
     _____________                     ___________________
cas               \___________________/                   \___________

# The Blitter

## Introduction

This section describes the graphics co-processor, or Blitter, of the SS4 system. It explains its modes of operation and how these are controlled by the programmer. The intention of this description is to allow a competent programmer to drive the Blitter hardware directly.

### Brief Introduction

The Blitter's operation can be viewed as a simple program:

read_command()
	for n=0 to outer_count
		read_parameters()
		for m=0 to inner_count
			if SRCEN then read_source()
			if DSTEN then read_destination()
			write_destination()
		next m
	next n

Various control bits in the blitter program control the width of the write, the data that is written, the way in which the addresses are updated both within the inner and outer loops, and can disable the write under certain conditions.

The program below will draw a white square on the screen, using the blitter. It assumes that the code is loaded at 1000:0000, and that the screen base address is at 0000:0400 and is a 256 byte wide 8-bit per pixel screen. The logical colour if FF and it is assumed this is white.

; WARNING - currently this program is untested

include	SS4.INC 
code_seg	segment	para	public 
		assume	cs:code_seg,ds:code_seg 
main		proc	far 
		mov	ax,offset bprog		; program address low word
		out	BLPROG0,ax 
		mov	ax,00001h			; program address high word
		out	BLPROG1,ax
		mov	ax,00111h			; command 
		out	BLCMD,ax 
		ret 
		EVEN 
bprog		DW	0,0				; source address (not used) 
		DW	02024h,0			; destination address 
		DW	0C040h			; outer count (block height) and LFU
		DW	00040h			; inner count (block width) 
		DW	0				; source step
		DW	0C0h				; destination step (256 byte screen width)
		DW	00FFh				; pattern (block colour) 
		DW	0				; next command (stop) 
main		endp 
code_seg	ends 
	end	main

## The Blitter

The Blitter is a co-processor of the SS4 system, whose purpose is to perform graphics creation and animation as fast as possible (limited by the memory bandwidth). It can perform arbitrarily long sequences of graphics operations by reading new command sets from memory.

While it is performing any graphics operations, the Blitter becomes a bus master, and denies the CPU any bus activity whatsoever. This is reasonable behaviour because the Blitter is being used to perform operations that the CPU would otherwise have performed, and is therefore speeding up program operation. This also removes the need for any asynchronous control programming for blitting operations, and the need for any interrupt generation hardware in the Blitter.

However, to allow real-time programming of either of the other two processors, the Blitter will suspend its operation and grant the bus to the DSP and Compact Disk DMA channels if they require a DMA transfer. It will also suspend itself and give up the bus to the CPU if an interrupt occurs. During any of these the current operation is suspended but will restart when the interrupt goes away (the interrupt signal goes inactive) or when the DSP DMA access completes.

The Blitter may be operated in a variety of modes to perform graphics and block move operations. These are described in the sections that follow.

## Architecture

This section discusses the architecture of the Blitter, so that its modes of operation can be understood.

### Overview

The internal architecture of the Blitter is best considered as three, largely separate, blocks. These are the Data Path, the Address Generator, and the Sequencer.

The Sequencer acts in software terms as the program that the Blitter runs, with two for loops and a couple of procedures. This program is fixed, although various parts of its operation are conditional on flags in the Blitter command, and the loop counts are also part of the command.

The address generator contains three address registers. These are the program address used to fetch Blitter commands, and the source and destination address registers. It also contains an ALU with an associated step register to update addresses, and a multiplexer to generate the output address.

The data path contains three data registers - the source data, destination data and pattern data registers. It also contains a versatile comparator to allow intelligent blitting operations, and a logic function unit to generate the output data.

These sections are described below.

## The Data Path

The Data Path contains three data registers, and two data manipulation blocks. These are the Logic Function Unit, which can combine the contents of the data registers in a number of useful ways to produce the output data; and the comparator, which can perform certain comparisons on the data to inhibit write operations, and optionally stop Blitter operation.

The data path can handle data in four quanta, 32, 16, 8 and 4 bits. Long-words (32 bits) are used when performing fast block moves and fills, and pixels (16, 8 or 4 bits) may be manipulated using all the Blitter modes, such as line-drawing, multiple plane operations, character painting and so on.

µ §

Figure - Data Path

### Data Path Width

The majority of the data path is sixteen bits wide as this is the maximum pixel size. However the source data register is thirty-two bits wide, and the top sixteen bits of this are used to produce the top sixteen bits of the data written in thirty-two bit mode, regardless of the LFU mode.

### Registers

There are two sixteen bit wide and one thirty-two bit wide data registers. These are the Pattern, Destination and Source Data registers respectively.

The Source and Destination data registers are loaded from the Source and Destination addresses in memory when the corresponding read cycles are enabled in the Inner Loop (see below). However, all three data registers are loaded at the start of Blitter operation with the pattern data, and this may be used as an additional source of data either in producing the output data or in the comparator. For example it may be a mask, a pattern for writing, a reference value, and so on. (The pattern data is loaded into both words of the source data register)

### Logic Function Unit

The Logic Function Unit (LFU) generates the output data, which is written to the destination in memory. It can perform any logical combination of the source and destination pixels (in this case, `source' data may be selected from either of the source or pattern data registers).

It does this by selecting any of the four Boolean minterms of the two sets of input data from the data registers, and generating the logical OR of the selected terms. This allows any logical combination of the of input data, there are therefore sixteen possibilities.

The minterms of a set of logical inputs are the set of AND terms given by each of the inputs combined together in all combinations of complimented and normal. Therefore the minterms of two inputs A and B are:

A  AND      B
	     A  AND (NOT B)
	(NOT A) AND      B
	(NOT A) AND (NOT B)

Any Boolean combination of A and B can be given by choosing a suitable set of minterms to OR together.

In thirty-two bit mode, the LFU will normally be set to produce source data, as it is only sixteen bits wide. The top sixteen bits written in a long-word write are always derived from the top sixteen bits of the source data register.

### Comparator

The comparator can perform a variety of comparisons on the data in the source, destination and pattern data registers. If its comparison conditions are met then it generates an inhibit signal. This is used to inhibit a write operation, and optionally to stop blitting operation.

The comparator may be used to provide a pixel plane effect, to give transparent colours, for collision detection and memory search operations, and as an aid to character painting.

### Comparator Pixel Plane Operation

Multiple plane operation is supported by assigning a plane number to every pixel. This mode is only applicable to four and eight bit pixels. In eight-bit pixel mode two of the eight bits are used giving two or four planes, in four-bit pixel mode one of the four bits is used giving two planes.

The bits are allocated within the byte thus:

| Bit | Plane Function |  |
| --- | --- | --- |
|  | 8 bit pixel | 4 bit pixel |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  | Plane bit |
| 4 |  |  |
| 5 |  |  |
| 6 | Plane bit 0 |  |
| 7 | Plane bit 1 | Plane bit |

The comparator can produce an inhibit output if the plane number of the destination data is equal to, not equal to, or greater than the plane number of the source data, or any combination of these. This means that data being written onto the screen can be masked by data already present in a different plane.

### Comparator Pixel Value Comparison

The comparator can produce an inhibit output if the entire source pixel is equal to or not equal to the destination pixel. This may be used, for example, for searching memory for a particular value; and, more importantly, for designating a colour to be transparent, and holding the transparent colour value in a data register. This applies to sixteen, eight or four bit pixels.

### Comparator Bit to Pixel Expansion

This comparator operation allows bit to pixel expansion of data, used for example for character painting. In this mode the comparator selects a bit of the source byte based on the value of the inner counter (refer to the Sequencer Description), and inhibits the write operation if this bit is zero.

### Handling of Data in Different Pixel Sizes

The Blitter makes provision for handling three screen pixel resolution modes. These are sixteen bit mode, where each word corresponds to one pixel; eight bit mode, where each byte corresponds to one pixel; and four bit mode, where each byte corresponds to two pixels.

In eight and sixteen bit pixel mode the data path is handling one pixel at a time, and this makes its operation straightforward. In four pixel bit mode, however, only half of the byte that is read from or written to is the current pixel so certain additional requirements are placed on the data path.

In a four-bit mode write operation, unchanged destination data is written to the half of the data byte that does not correspond to the current pixel. This means that destination reads must always be enabled in four-bit mode (set control bit DSTEN). This is because there is no provision for writing less than one byte into main memory.

It is also possible that the source four bit pixel address and the destination four bit pixel address point at different halves of the corresponding bytes of RAM. If this is the case, a shifter swaps the two halves of the source data.

In four bit mode, the two nibbles of the pattern byte should normally be set to the same value.

Note that the pixel width programmed in the Blitter does not have to match the display width, and the most efficient way of moving large amounts of data is thirty-two bit mode, noting that it has to be long-word aligned (and the system has to be fitted with thirty-two bit RAM).

## The Address Generator

The address generator contains three address registers, an increment or step register, an address adder, and an address output multiplexer.

µ §

Figure - Address Generator

### The Address Registers

There are three address registers holding the source address, the destination address, and the program address.

Each of these registers contains twenty-four address bits, referred to as bits 0 to 23, allowing the Blitter to address up to sixteen megabytes. In addition to this the source and destination address registers contain a nibble bit used in four-bit pixel mode, designated bit -1.

The Program address register holds the address that the program is fetched from, and is incremented by one word each time a memory cycle is performed using it. This register is always even, so always write 0 to bit 0.

The Source and Destination Address registers are updated after each cycle, and at other times, using an adder that allows them considerable flexibility in the objects to which they refer.

All source and destination address updates may be performed optionally on just the bottom sixteen to nineteen bits of the address register. This means that the Blitter will then effectively operate in 64K, 128K, 256K or 512K pages. In this mode if an address overflows in a page, it will wrap, and the overflow (or underflow) will be lost.

### The Address Adder

The address adder is a twenty-five bit wide adder used to update addresses. It allows either a constant value of 0.5, 1 or 2, or a variable stored in one of the step registers, to be added to an address value. It can also subtract the same values. The twenty-fifth bit is the nibble part of the address.

Note that an increment of one pixel has a different effect on the address depending on the screen resolution set, and that this is automatically catered for.

All address registers are updated automatically at the end of the appropriate memory cycles, source read for the source address register, and destination write (even if inhibited) for the destination address register.

Addresses can be made to wrap vertically by the SWRAP and DWRAP bits in the blitter command, and horizontally by the SLWRAP and DLWRAP bits in the blitter control register.

### The Address Output Multiplexer

The address output multiplexer provides the external address to the system memory. It provides three types of address, these are the Source Address, the Destination Address and the Program Address. These are derived directly from the corresponding address registers.

### Line Drawing Address

When drawing lines is being performed, the address registers are used in a different way from normal. The Destination Address register is used as the line draw address, and the Source Address register and Step register are used as Delta One and Delta Two respectively. During line drawing Delta Two is subtracted from Delta One, and the borrow output produced is used to determine what is added to the Destination Address register. See the section on line-draw below.

## The Sequencer

The Sequencer and controller section controls the operation of the other sections of the Blitter, and therefore governs the overall operation of the device.

The flow of control is best considered at two main levels. There is an outer loop governing the overall flow of control, and within that three subsections, the inner loop, the parameter read procedure and the command read procedure.

µ §

Figure - Control hierarchy

This might also be viewed as a simple program, e.g.

read_command()
	for n=0 to outer_count
		read_parameters()
		for m=0 to inner_count
			if SRCEN then read_source()
			if DSTEN then read_destination()
			write_destination()
		next m
	next n

(This ignores a lot of the Blitter control bits)

The blocks in the diagram above represent state machines within the Blitter, and various control bits within the Blitter command affect their operation. To set these control bits correctly requires some understanding of the flow of control within the Blitter, and this is discussed below.

### The Inner Loop

The inner loop performs the actual blitting or line-drawing operation.

µ §

Figure - Blitter Inner Loop Flow Diagram

An Inner Loop cycle can contain up to three memory cycles. These are a read from the source address, a read from the destination address, and a write to the destination address. All three cycles are optional.

If the loop type includes a source read, or a source read and a destination read, then the comparator inhibit mechanism is tested before the destination write occurs. This allows the write cycles to be bypassed when the comparator inhibit conditions are met.

Under these circumstances it is possible to have the current operation cease and control returned to the CPU. The program may then examine the address registers to determine where the inhibit has occurred, so that collision detection may be performed; and may determine whether to resume operation or abort it.

The Inner Loop is repeated until the inner counter reaches zero. This is an ten-bit counter, and so can be any number of times from 1 to 1024.

### Collision Detection

As can be seen in the diagram above, the Blitter makes provision for collision detection by allowing operation to stop when a comparator write inhibit occurs. When this happens, control returns to the CPU, which may then examine the internal state of the Blitter to determine what has caused the collision.

At this point, the CPU may choose to allow the Blitter to resume the operation it was performing, or may reset it back to its idle state. Either a reset or a resume command must be issued before the Blitter may be used for another operation.

Note that while the Blitter is in this suspended state a new value may be written to the command register, so that the collision stop mechanism may be disabled.

### Inner Loop Execution Times

Normally the inner loop will execute at a rate determined only by the cycle time of the memory being accessed, with no gaps between memory cycles. The only exception to this is an inhibited write cycle, in which case two clock cycles are required for destination address updating.

### The Parameter Read Procedure

The parameter read procedure is a very straightforward sequence that loads up a new set of parameters to the inner loop. It reads, in sequence, the inner loop counter value, the step register values, and the pattern value; used to pre-set the data registers.

The inner count is the number of times the Inner Loop is repeated; the step registers are used for address incrementing, and the pattern is used for data manipulation.

The parameter read procedure is called as part of the command read operation at the start of a Blitter operation; and is also called if required during a blitting operation (the PARRD control bit determines this). These extra parameter reads occur between passes through the Inner Loop, to allow various parameters of the loop to be altered. This allows such operations as irregular shape painting and run-length encoded data expansion.

### The Command Read Procedure

The command read procedure is used to start a new blitting operation. The inactive, reset state of the Blitter is part of this procedure. It looks like this:

µ §

Figure - Blitter Command Read Loop Flow Diagram

The Stopped state of the Command read loop represents the normal inactive state of the Blitter. From this state a Command register I/O write is performed to start the Blitter, usually preceded by a write to the program address register. The full set of operational parameters is loaded from the program count address, which is auto-incremented, and control passes out of the command read loop.

When a blitting operation is completed, a new command is read from the program count address, and if this command leaves the Blitter in Run mode, then a new set of parameters is loaded and another operation is started. Otherwise the Blitter enters its Stopped state and returns the bus to the CPU.

This mechanism allows the Blitter to perform arbitrarily long sequences of graphics commands without requiring any processor intervention. This is extremely useful, as processor I/O write cycles are relatively slow in comparison to Blitter memory reads.

### The Outer Loop

The Outer Loop controls the overall operation of the Blitter, including starting off the three inner loops described above. Its flow of control looks like this:

µ §

Figure - Outer Loop Flow Diagram

Normal operation starts on exit from the command read loop. The parameter read loop then entered to read the first set of parameters, and then the inner loop is entered, with the inner counter being loaded to its initial value before the start of operation.

The Outer counter is then decremented, and if it is zero, then the command read loop is entered; then either or both of the source address and destination address register may be updated with the contents of the step register. The parameter read loop may then be optionally entered to update various inner loop parameters, before the inner loop is entered again.

The two loops allow the Blitter it to cope with two-dimensional screen structures, with the outer loop address register updates moving screen address pointers onto the start of the structure on the next line. The parameter read loop adds flexibility by allowing the screen structure parameters to be altered on a line by line basis.

## Memory Interface and Interrupts

All the memory cycles are controlled by a memory interface state machine. This controls the cycle timing generation, and the bus arbitration.

The blitter will take over the bus from the host processor for the duration of a blitter command sequence. This is subject to the bus hand-over latency discussed above, but as soon as the blitter is granted the bus its operation will start.

The memory interface will give up the bus to the DSP or the compact disk read channel as soon as one of these requests the bus, pausing only to complete any current memory cycle.

Interrupts will also cause the blitter to suspend operation, unless masked in the blitter control register. The blitter detects the state of the interrupt line itself, and uses this to suspend operation. Operation will resume as soon as the interrupt line is cleared, which occurs when the host processor write to the ACK port occurs. As this may not necessarily be the end of the interrupt service routine, programmers should be wary of stack crawl, and should normally keep interrupts disabled during a service routine.

The blitter will resume operation as soon as the interrupt line is cleared without any intervention from the host processor.

Note that the blitter only responds to the internal interrupt sources, i.e. the video, analogue inputs, and compact disk interrupts. Any external interrupt source has no effect on the blitter.

## Modes of Operation

This section describes some of the modes of operation of the Blitter, and how they should be programmed. An understanding of the previous section describing the mechanisms of the Blitter is assumed.

### Simple Memory Fill and Copy Operations

The simplest operations are those involving copying one block of memory to another, and filling a block of memory with a pre-defined value. These operations can be performed on linear parts of memory, and on arbitrary screen rectangles.

The destination data register is used as the address of the memory being modified, and the source address register is used as the address of the data being copied, if it is a copy operation.

### Handling of Data

The output data being given by the Logic Function Unit will be selected as Pattern data in the case of a memory fill, or as Source data in the case of a memory copy. For simple operations, the comparator control register will be set to zero.

### Addressing and Counters

When the operation is to be performed on linear areas of memory, most of the address control bits will be set to zero. The step register is not used, and the only requirement is to determine whether the copy will be made with the address incrementing or decrementing, and setting DSIGN and SSIGN appropriately. Note that the initial value placed in the address register should be the bottom of the area to be operated on if the sign bit is not set, and at the top if it is. In both cases, the first pixel read or written will be the first address. The length of the operation will be placed in the inner counter, and the outer counter set to one.

If the block being operated on is very large, then the inner and outer counters may both have to be used, and the number of pixels operated on will be given by the product of the inner and outer counter values.

When either or both of the source and destination data are rectangles rather than linear areas, then the inner counter will contain the rectangle width, and the outer counter the rectangle height. The appropriate step register is set to the address increment from the right hand side of the rectangle round to the left hand side on the next line. The SRCUP and DSTUP bits are set according to whether the source or destination are rectangles.

### Inner loop mode control

In eight or more bits per pixel mode, neither SRCEN nor DSTEN will be used for memory fill, bit SRCEN should be set for memory copy. In four-bit pixel mode DSTEN must always be set as well, so that a destination read is performed to avoid corrupting the other pixel. Note that this will be slower.

## Line Drawing

Line drawing is based on a simple D.D.A. (Digital Differential Analyzer) algorithm. The basis of this is that for a given line one of the X-address or Y-address is always incremented for every pixel drawn, while the other one is incremented if a suitable arithmetic condition is met. This produces a line like this:

µ §

The algorithm used by the Blitter computes the arithmetic condition that causes the conditional increment by repeated subtraction of the smaller of dx or dy from a working value, with the larger being added back when underflow occurs - effectively division to give the gradient.

The notation dx refers to the distance along the X-axis that the line corresponds to. This is given by |(x1 - x2)| where x1 and x2 are the X co-ordinates of the end points - the vertical bar notation means the magnitude or modulus of their difference.

### Computation of line-draw arguments

If a line is being drawn from (x1,y1) to (x2,y2), then:

dx = |(x1-x2)|

dy = |(y1-y2)|

From these d1 is given by the larger of dx and dy, d2 by the smaller. Then, for each pixel drawn, d2 is subtracted from a working value, which is initially set to d1/2, and the sign of the result of this subtraction (indicating underflow) is the arithmetic condition for the conditional part of the screen address update. When this underflow occurs, the original value of d1 is added back to the working value.

It can be seen that the ratio of dx to dy will give the frequency with which this underflow and adding-back occurs. The ratio between them is, of course, also the gradient of the line (Q.E.D!).

### Creating a line draw Blitter command

These values are used to set the Blitter command as follows:

·	The start point of the line (x1,y1) is the destination address

·	d1 is placed in bits 10 to 19 of the source address register - this is the stored original value, and d1/2 is placed in bits 0 to 9 - this is the working value.

·	d1 also gives the inner counter value, although d1+1 should be used if both end points of the line are to be drawn.

·	d2 is placed in the source step register.

·	If dx > dy then the YFRAC flag is set, otherwise it is cleared.

·	SSIGN gives the sign of X-address updates, DSIGN the sign of Y-address updates.

### Handling of Data

As all the registers in the address section are occupied in computing the line address, the Blitter has no ability to move data from somewhere else when drawing lines. Therefore the data written at the line address has to be given either directly by the pattern data or by a combination of the pattern register and the data already there, according to the L.F.U.

The implication of this is that SRCEN should not be set, for it would produce seemingly random data!

### Addressing and Counters

The addressing has largely been discussed above. The inner counter is set to the length of the line, and the outer counter will be set to one.

### Mode control

In eight or more bits per pixel mode, DSTEN need not be set, unless used for read-modify-write operation. In four bits per pixel mode, DSTEN must always be set, so that a destination read is performed to avoid corrupting the other pixel.

## Character Painting

The Blitter has the ability to paint characters on the screen as a single operation. Character painting as far as the Blitter is concerned involves painting a rectangular area up to 8 pixels wide and of arbitrary height. The pixels in this area are either written to or left unchanged according to a bit pattern.

This mode of use is not restricted to character painting, but may be used for expanding any graphics stored as a monochrome bit plane.

### Character Font

The source register addresses the bit pattern, normally part of the font, where each byte corresponds to one row of the character. Therefore Blitter fonts may be up to 8 pixels wide. Wider fonts may of course be used, but these will require more than one Blitter operation to paint a character.

The data is arranged with the bit corresponding to the left-most pixel in the least significant bit, and the top of a character at the lowest address. If the data is less than 8 pixels wide, then the least significant bits of the font data are not used. For example, the font entry for the letter R in a 6x10 font would be something like this:

| Byte | Pixels set | Binary | Hex |
| --- | --- | --- | --- |
| 0 | xxxx | 01111000 | 78 |
| 1 | x x | 10001000 | 88 |
| 2 | x x | 10001000 | 88 |
| 3 | x x | 01001000 | 48 |
| 4 | xxx | 00111000 | 38 |
| 5 | x x | 00101000 | 28 |
| 6 | x x | 01001000 | 48 |
| 7 | x x | 10001000 | 88 |
| 8 | x x | 10001000 | 88 |
| 9 |  | 00000000 | 00 |

### Mode Control and Screen Addressing

The destination address register is used to address the area of the screen to which the character is to be painted. Normally this area has been cleared to the required background colour by a previous Blitter operation. The destination address is initialised to the top left hand corner of the character.

The character to be painted is a rectangle, and therefore the destination addressing is programmed correspondingly. The Inner Counter is set to the width of the character and the Outer Counter to its height. The destination step register is set to be the screen width less the width of the character. The DSTUP bit is set to allow the destination address to be updated between passes through the inner loop.

Inner loop control bits DSTEN and SRCENF are set, character painting being the reason for the existence of SRCENF. This allows the font byte for each row to be read just once.

The comparator is used to control the painting of pixels, and so the CMPBIT control bit is set, to enable its bit to byte expansion mechanism.

### Handling of data in character paint

The colour to be painted is set as the pattern, and this will normally be held in the pattern data register. In four-bit pixel mode, DSTEN will be set, and the destination data register will hold the read value so that the other half of the byte may be written back undisturbed. The source data register holds the font pattern, as mentioned above.

## Rotate Mode

This mode uses the shading ALU, but instead of producing three DDA based data values, it produces two DDA based address values, X and Y. Normally, these are used to traverse a source data field at arbitrary angles and rates so that the destination data corresponds to a scaled and/or rotated version of them.

The red value generator gives the X value and the green value generator gives the Y value. The blue value generator is not used, and clearly shading cannot be used in conjunction with this mode.

As rotation requires higher accuracy than shading, four extra integer bits are added to the X and Y values, and these are set up in rotate registers 0 and 1. All calculations are performed to ten point ten bit accuracy.

As with shading, the delta values are added to X and Y after each pixel is drawn in the inner loop. The step values are added in the outer loop, and both the SRCUP and DSTUP flags must be set for them to be added. The delta and step values may be either positive or negative, and no adder saturation occurs, unlike shading mode.

Normally, rotation and scaling are performed by setting the destination address pointer to perform a normal raster scan over the destination rectangle, while the source pointer traverses over the source data at a suitable gradient and rate. This ensures that the destination data is contiguous, and that no more blits than necessary are required. The source data should be surrounded with a suitable transparent colour if the target area is not rectangular.

The address is formed as follows (noting that if PSIZE1 is set the bottom bit is masked to zero, so the red value has one more fractional bit and one less integer bit):

| Width: | 256 | 512 | 1024 | 2048 |
| --- | --- | --- | --- | --- |
| 0 | RED0 | RED0 | RED0 | RED0 |
| 1 | RED1 | RED1 | RED1 | RED1 |
| 2 | RED2 | RED2 | RED2 | RED2 |
| 3 | RED3 | RED3 | RED3 | RED3 |
| 4 | RED4 | RED4 | RED4 | RED4 |
| 5 | RED5 | RED5 | RED5 | RED5 |
| 6 | RED6 | RED6 | RED6 | RED6 |
| 7 | RED7 | RED7 | RED7 | RED7 |
| 8 | GREEN0 | RED8 | RED8 | RED8 |
| 9 | GREEN1 | GREEN0 | RED9 | RED9 |
| 10 | GREEN2 | GREEN1 | GREEN0 | 0 |
| 11 | GREEN3 | GREEN2 | GREEN1 | GREEN0 |
| 12 | GREEN4 | GREEN3 | GREEN2 | GREEN1 |
| 13 | GREEN5 | GREEN4 | GREEN3 | GREEN2 |
| 14 | GREEN6 | GREEN5 | GREEN4 | GREEN3 |
| 15 | GREEN7 | GREEN6 | GREEN5 | GREEN4 |
| 16 | GREEN8 | GREEN7 | GREEN6 | GREEN5 |
| 17 | GREEN9 | GREEN8 | GREEN7 | GREEN6 |
| 18 | ROTA18 | GREEN9 | GREEN8 | GREEN7 |
| 19 | ROTA19 | ROTA19 | GREEN9 | GREEN8 |
| 20 | ROTA20 | ROTA20 | ROTA20 | GREEN9 |
| 21 | ROTA21 | ROTA21 | ROTA21 | ROTA21 |
| 22 | ROTA22 | ROTA22 | ROTA22 | ROTA22 |
| 23 | ROTA23 | ROTA23 | ROTA23 | ROTA23 |

The rotate control registers are described below.

## Blitter Command Format

A Blitter command is given as a table of data in memory. The Blitter loads the contents of this table into its internal registers and performs the specified operation. The Blitter will read successive sets of commands until a stop instruction is read into the command register.

The Blitter program address must be set up before the command word is issued. The Blitter program address is given by the program address registers, which together give the full twenty-four bit address. The program must lie on a word boundary.

A full table of Blitter command data starts with a command word. However the first Blitter command in a sequence has its command word written into the command register by a CPU I/O cycle, and so starts reading the command data from the second word (Word 1); and similarly the last Blitter command need consist of no more than a command word with the RUN bit clear. A Blitter command takes the form:

µ §

If the SHADE bit is set, then nine additional words are fetched, as follows

µ §

Figure - Blitter command summary

### Command Bits

| RUN | causes the Blitter to start operation. It is used when writing to the command register as an I/O port to start the Blitter reading a command. If the Blitter loads a command with RUN set to zero, as part of a command read, then operation ceases. |
| --- | --- |
| COLST | causes operation to stop if a collision (write inhibit) occurs. From this point the current operation can be resumed or aborted, and various internal registers may be read. (See comparator control bits). |
| PARRD | requires the Blitter to read a new parameter set (that is the inner count and comparator bits, the pattern register, and the step values) from the program counter address, every time the inner loop exits and the outer count has not reached zero. |
| SRCUP | requires the contents of the step register to be added to the source address on exit from the inner loop if the outer count has not reached zero. |
| DSTUP | requires the contents of the step register to be added to the destination address on exit from the inner loop if the outer count has not reached zero. |
| SRCEN | enables a source address read in the inner loop. This will also cause the source address register to be incremented according to the pixel size. |
| DSTEN | enables a destination address read in the inner loop. This does not affect the destination address register which is incremented as part of the destination write cycle. |
| SRCENF | is a special case of SRCEN and causes the source address to be read when the inner loop is first entered, but not subsequently. This is relevant to character paint mode. SRCENF has no effect if SRCEN is set. |
| PSIZE0-1 | elects the pixel size, 0-3 corresponding to 4, 8, 16 and 32 bits. 32 bits is for data moves in a 32-bit bus system only. |
| WIDTH0-1 | selects the screen width, in bytes, 0-3 corresponding to 256, 512, 1024 and 2048 bytes. |
| LINDR | sets line-drawing mode. This mode uses both the source and destination address registers to generate the line-draw address, which may be used for both reading and writing. |
| YFRAC | indicates which of the X and Y addresses has the fractional increment in line-drawing mode. It is set if the Y address has the fractional increment. |
| PATSEL | is used to select the pattern data register to replace the source data register as the source input to the LFU. This is relevant to character painting, where the source data register will contain the font data, and the pattern data register will contain the ink colour. |
| SHADE | enables output from shading ALU as write data. Only valid for 8 and 16 bit pixels. |

### Source Address Register and Control Bits

The Source Address Register is used as a pointer to a source of data for blitting operations. Bits SRCA0 to SRCA23 correspond directly to address lines 0 to 23. The SRCA-1 bit is used as a nibble address in four bits per pixel mode.

The source address register is also used as d1 and as the division workspace in line-drawing mode. Refer to the description of line-drawing mode.

| SWRAP | causes source address updates to wrap on a programmable boundary, as opposed to running linearly through memory. |
| --- | --- |
| SWRAP0-1 | control the size of the SWRAP function, which makes the source address pointer wrap vertically, 0-3 corresponding to 64K, 128K, 256K and 512K screens. |
| SRCCMP | selects the source data register as the source input to the comparator. If it is clear, the pattern data register is used. |
| SLWRAP | makes the source pointer wrap within the line width for inner loop updates. |
| SSIGN | is the sign used when updating the source address. Setting it causes the source address to be decremented rather then rather then incremented. It makes X negative in line-drawing. |

### Destination Address Register and Control Bits

The Destination Address Register is used as a pointer to the destination of the data in blitting operations. Bits DSTA0 to DSTA23 correspond directly to address lines 0 to 23. The DSTA-1 bit is used as a nibble address in four bits per pixel mode.

| DWRAP | causes destination address updates to wrap on a programmable boundary, as opposed to running linearly through memory. |
| --- | --- |
| DWRAP0-1 | control the size of the DWRAP function, which makes the destination address pointer wrap vertically, 0-3 corresponding to 64K, 128K, 256K and 512K screens. |
| DSTCMP | selects the destination data register as the source of destination data to the comparator. If it is clear then the pattern data register is used. |
| DLWRAP | makes the destination pointer wrap within the line width for inner loop updates. |
| DSIGN | is the sign when updating the destination address. Setting it causes the destination address to be decremented rather then incremented. It makes Y negative in line-drawing. |

### Logic Function Unit Control Bits

| LFU 0-3 | The logic function unit (L.F.U.) controls the data that is written in a destination write cycle. The L.F.U. allows any logical combination of the source and destination data. This is achieved by each of the L.F.U. bits selecting one of the Minterms, with the output being given by the OR of the selected terms. The correspondence between bits and Minterms is LOG0 NOT source AND NOT destination LOG1 NOT source AND destination LOG2 source AND NOT destination LOG3 source AND destination There are therefore sixteen possibilities. |
| --- | --- |

### Outer and Inner Counters

| OLCNT 0-9 | are the outer loop counter initial value. This is used to initialise into the outer loop counter. Note that setting the outer count to zero will produce 1024 outer loop operations. |
| --- | --- |
| ILCNT 0-9 | are the inner loop counter initial value. This is loaded into the inner loop counter every time the inner loop is entered. Note that setting the inner count to zero will produce 1024 operations. |

### Comparator Control Bits

| CMPPLN | enables plane mode where the three comparator functions operate on the plane number bits as opposed to the entire pixel. |
| --- | --- |
| CMPEQ | causes the comparator to inhibit an inner loop write, if in plane mode the priority of the destination pixel is equal to the plane priority of the source pixel, or if the entire pixel is the same if not in plane mode. |
| CMPNE | causes the comparator to inhibit an inner loop write, if in plane mode the priority of the destination pixel is not equal to the plane priority of the source pixel, or if the entire pixel is not the same if not in plane mode. |
| CMPGT | only operates in plane mode, and causes the comparator to inhibit the write if the plane priority of the destination pixel is greater than the plane priority of the source pixel. |
| CMPBIT | is used to give a bit to byte expansion scheme. It causes the comparator to generate an inhibit by selecting a bit of the source data register using the inner counter, and generating an inhibit if the bit selected is a zero. The selection is given by 8 in the inner counter selecting bit 0, 7 selecting bit 1, 6 bit 2, and so on. |

### Step Registers

| SSTEP 0-11 | are the source step register value, and is a signed integer - this sign override the SSIGN setting. The step is used to increment or decrement the source address by a number other then 1, so that non-contiguous screen data may be handled in single operations. The source step register is also used in line drawing mode to hold d2. |
| --- | --- |
| SSTEP -1 | is the least significant bit of the step. It is used a the nibble bit of the step when in four bits per pixel mode. |
| DSTEP 0-11 | are the destination step register value, and is a signed integer - this sign override the DSIGN setting. The step is used to increment or decrement the destination address by a number other then 1, so that non-contiguous screen data may be handled in single operations. |
| DSTEP -1 | is the least significant bit of the step. It is used a the nibble bit of the step when in four bits per pixel mode. |

### Program Address Register

The Program Address points to the source of blitting operation commands. Data is read from it sequentially upwards through memory. It must always be even (i.e. Blitter programs must lie on word boundaries). Register 0 corresponds to address bits 0-15, and register 1 to address bits 16-23 in bits 0-7.

## Blitter I/O Registers

The Blitter allows some of the above registers to be visible in the CPU I/O space. In addition to these, some Blitter status and control bits are accessible to the CPU.

The Blitter has seven word wide read registers and four word wide write registers. Any unused bits in the write registers should be written with a zero.

The IO register appear starting from IO address 40h. These registers are also available in the memory map, principally so the DSP can access them, starting at the same offsets as IO but at base address F10400h (i.e. subtract 40h and add F10400h to get the memory address).

### BLITDST0	Blitter Destination Register 0	40h	RO

Bits 0-15 correspond to bits 0-15 of the destination address register.

### BLITDST1	Blitter Destination Register 1	42h	RO

| Bit | Name | Description |
| --- | --- | --- |
| 0-7 | DA16-23 | Destination address top eight bits |
| 8-14 | unused |  |
| 15 | DAM1 | Destination address nibble part |

### BLITSRC0	Blitter Source Register 0	44h	RO

Bits 0-15 correspond to bits 0-15 of the source address register.

### BLITSRC1	Blitter Source Register 1	46h	RO

| Bit | Name | Description |
| --- | --- | --- |
| 0-7 | SA16-23 | Source address top eight bits |
| 8-14 | unused |  |
| 15 | SAM1 | Source address nibble part |

### BLITINN	Blitter Inner Counter	48h	RO

| Bit | Name | Description |
| --- | --- | --- |
| 0-9 | INNER0-9 | Inner counter value |
| 8-15 | unused |  |

### BLITOUT	Blitter Outer Counter	4Ah	RO

| Bit | Name | Description |
| --- | --- | --- |
| 0-9 | OUTER0-9 | Outer counter value |
| 8-15 | unused |  |

### BLITSTAT	Blitter Status	4Ch	RO

This register gives a variety of blitter status information

| 0 | INHIB0 | Comparator plane priority greater than condition is met |
| --- | --- | --- |
| 1 | INHIB1 | Comparator plane priority equal condition is met |
| 2 | INHIB2 | Comparator plane priority not equal condition is met |
| 3 | INHIB3 | Comparator pixel equal condition is met |
| 4 | INHIB4 | Comparator pixel not equal condition is met |
| 5 | INHIB5 | Comparator bit to pixel condition is met |
| 6-12 | unused |  |
| 13 | RUN | Blitter is currently active, or operation is suspended by a CPU interrupt or a collision stop. |
| 14 | ISTOP | Blitter has stopped for a CPU interrupt |
| 15 | STOP | Blitter has stopped because of a collision detection |

### BLITPC0	Blitter Program Address 0	40h	WO

This register is loaded with bits 0-15 of the Blitter program address. Note that bit 0 is always zero (Blitter programs must lie on word boundaries).

### BLITPC1	Blitter Program Address 1	42h	WO

This register is loaded with bits 16-23 of the Blitter program address in bits 0-7. The other eight bits must always be set to zero.

### BLITCMD	Blitter Command Register	44h	WO

This register corresponds to word zero of the blitter command, and is used to set up the command when the blitter is started. Blitter DMA will then start from word one of the command.

### BLITCON	Blitter Control Register	44h	WO

| Bit | Name | Description |
| --- | --- | --- |
| 0 | IMASK | Interrupt stop mask, masks interrupts from the Blitter's bus control unit when set, with the result that the Blitter will not stop when an interrupt occurs. |
| 1 | RESUME | Resume operation after collision, is used to re-start the Blitter after a collision has been detected (COLST bit is set). The Blitter will resume the operation which it has suspended. Note that it is possible to re-program the Blitter command register while the Blitter is in the Collision Stop state, so the COLST bit, in particular, may be changed. |
| 2 | RESET | Reset to quiescent after collision, is used to abort the operation the Blitter was performing when a collision stop has occurred. |
| 3-15 | unused |  |

Note that after a Blitter collision stop either a resume or a reset should be issued to the Blitter.

### DSTROT0	Rotate Register 0	4Ah	WO

| 0-3 | XI6-9 (RED6-9) | The top four bits of the integer part of the X address, the bottom six bits of this ten bit value are the integer part are the red integer value. |
| --- | --- | --- |
| 4-7 | XINC6-9 | The top four bits of the integer part of the X increment, the bottom six bits of this ten bit value are the integer part are the red integer value. |
| 8-11 | XSTP6-9 | The top four bits of the integer part of the X step, the bottom six bits of this ten bit value are the integer part are the red integer value. |

### DSTROT1	Rotate Register 1	4Ch	WO

| 0-3 | YI6-9 (GREEN6-9) | The top four bits of the integer part of the Y address, the bottom six bits of this ten bit value are the integer part are the green integer value. |
| --- | --- | --- |
| 4-7 | YINC6-9 | The top four bits of the integer part of the Y increment, the bottom six bits of this ten bit value are the integer part are the green integer value. |
| 8-11 | YSTP6-9 | The top four bits of the integer part of the Y step, the bottom six bits of this ten bit value are the integer part are the green integer value. |

### DSTROT2	Rotate Register 2	4Eh	WO

| 0 | DST_SHA | When this bit is set, the rotate address replaces the destination |
| --- | --- | --- |
| 1 | SRC_SHA | When this bit is set, the rotate address replaces the source address. |
| 2 | ROTATE | Sets rotation mode, as opposed to shading mode. |
| 10-15 | ROTA18-23 | The top bits of the rotate address. |

# The Digital Sound Processor

### Introduction

The DSP system in the SS4 System is a general purpose arithmetic processor with sufficient power to implement a high performance music synthesizer. Synchronous serial outputs are provided for  generation of stereo audio signals with sixteen bit precision and hence giving the sort of sound quality normally associated with Compact Disk technology.

The DSP is micro-programmable from the host CPU and the instruction set is sufficiently flexible to enable the user to program the device to fulfil many different functions that are considerably at variance with that of 'music synthesizer'. Such applications might include algorithmic speech generation, and audio analysis using fast Fourier transform techniques.

The DSP uses Harvard Architecture for maximum data throughput (separate program and data buses).

The ALU features a hardware 16 x 16 hardware multiply/accumulate as well as addition, subtraction and logical functions. There is also a separate serial divide unit, which generates one quotient bit per tick. The carry bit from the adder/subtracter is stored in a separate latch and can be either used either to propagate carry for multiple precision arithmetic or can be used for conditional instructions. All instructions may be made to be dependant on this bit being set.

Data transfers within the device (with the exception of internal transactions within the multiplier/accumulator) are all 16 bit.

## Architecture

The DSP is a very simple, very fast processor intended primarily for sound synthesis, but also capable of other computational tasks (such as graphics processing - 3D rotation, etc.). It executes all instructions in one processor cycle, and these occur at the system clock speed (20-25 MHz typically).

During sound synthesis, the DSP has its timing controlled by timers in the DAC interface. These DACs are double-buffered, and if a DAC write is about to cause overflow then operation is suspended until the buffer is empty. This allows the software to execute loops at sample rate, and as long as the average loop time is less than the sample period, then occasional loops can be up to twice as long.

As the loop may contain more instructions than will fit in the program RAM,  the DSP has an indexed addressing mode, which allows the same piece of code to act on several voices.

The DSP is a Harvard Architecture device, which means that the program and data RAM are quite separate, with cycles occurring in both RAM blocks at the same time. A one-cycle pipe-line is used, so that in each clock cycle an instruction is fetched, and the data transfer associated with the previous instruction takes place. (This has the odd effect that the instruction after a jump is executed).

µ §

Figure - Simplified DSP Diagram

This simplified diagram helps to show how the DSP works. In the first tick of the execution of an instruction the op code is read from the program RAM into the Instruction Decoder. In the second tick (during which the next instruction is read) a data transfer is performed either from memory to a register, or a register to memory.

Consider, for example, this sequence of instructions to perform an ADD:

MOV	X,operand1
	ADD	operand2
	MOV	result,AZ

The first instruction moves data from a location in data memory (which could be ROM, RAM, a register or a constant), to the X register shown on the diagram. The next instruction, the ADD, reads a second word of data from data memory, which is added to the contents of the X register by the ALU, and the result is loaded into the AZ register. The third instruction transfers the data from the AZ register back into data memory.

All DSP instructions involve either a memory to register transfer, or a register to memory transfer (or no transfer). Register to register transfers may in fact occur, as all the registers appear in the data memory space as well as being part of instructions.

### ALU

The ALU is a 16-bit Arithmetic Logic Unit, with the same function as a Texas Instruments 74181 device. Common arithmetic operations are encoded as instructions (ADD, ADC, SUB, SBC, AND and OR). However, by setting up the ALU mode bits a variety of useful (and some weird) arithmetic operations may be performed by the GAI instruction, GAI stands for general purpose arithmetic.

Note that the ALU carry flag means NOT borrow in the case of subtract operations, unlike some micro-processor ALUs.

### Multiplier

The multiplier is a second ALU which performs 16x16 signed/unsigned multiplies to yield a 32-bit result. In addition to this, it may also perform multiply-accumulate operations, where the product of the multiply is added to the previous result. The result is accumulated to 36-bits to allow for overflow.

Multiplier operations actually take two ticks, although the multiply instruction itself completes in one tick. This means that the instruction following a multiply (MULT), or a multiply-accumulate (MAC), may not involve the MZ registers or the X register.

### Divider

The division unit appears as a set of registers in the internal DSP space. It is capable of unsigned division on sixteen or thirty-two bit operands, and produces a quotient and a remainder.

### DMA Channel

The DMA channel appears as a set of registers in the DSP data memory space. These are two address registers, DMA0 and DMA1, and a data register, DMD.

A DMA transfer is initiated by writing an address to the DMA0 register. These DMA transfers have a latency period, which must be allowed to elapse before performing further DMA, this is discussed later. The DMA state machine is responsible for requesting the bus, and when it is granted performing the transfer, after which the bus is released.

Alternatively, a word may be written to DMA1 with the HOLD bit set. This will request the bus and retain it until this bit is cleared. This may be more efficient when performing successive multiple transfer, but is generally less efficient for single transfers as the DSP program cannot determine when the bus is granted, and therefore has to wait the maximum possible latency..

### Memory

DSP memory is generally visible both in the DSP's internal data address space, and in the host address space (using the intrude mechanism).

## Data Address Space

All undefined write bits in these registers must be set to zero, to avoid either enabling manufacturing test modes, or incompatibility with future enhancements.

Data space is organised as follows:

| Host | DSP | Function |  |
| --- | --- | --- | --- |
| F18000-1FF | 000-0FF | Map 0: 256 x 16 full wave two's complement SINE table Map 1: 256 x 16 data RAM |  |
| F18200-218 | 100-10C | Useful constant table, these are hard-wired for general use. |  |
| F18200 | 100 | 0000h |  |
| F18202 | 101 | 0001h |  |
| F18204 | 102 | 0002h |  |
| F18206 | 103 | 0004h |  |
| F18208 | 104 | 0008h |  |
| F1820A | 105 | 0010h |  |
| F1820C | 106 | 0020h |  |
| F1820E | 107 | 0040h |  |
| F18210 | 108 | 0080h |  |
| F18212 | 109 | FFFFh |  |
| F18214 | 10A | FFFEh |  |
| F18216 | 10B | FFFCh |  |
| F18218 | 10C | 8000h |  |
| F1821A-27E | 10D-13F | Reserved for future expansion. |  |
| F18280 | 140 | Intrude data register. This gives the data of the last (or pending) host intrude transfer. This is of little relevance to DSP programs, although it might conceivably be used as some sort of semaphore. |  |
| F18282 | 141 RW | Index register. This register is added to the address in the instruction when the indexed addressing mode is enabled for that instruction. This is particularly useful for writing procedure which will act over several data sets, or for indexing the Sine table. |  |
| F18284 | 142 RW | DMA address 0. This register contains the bottom sixteen bits of the DMA transfer address. A write to this register initiates the DMA transfer. |  |
| F18286 | 143 RW | DMA address 1 and DMA control. This register should be set up before initiating a DMA transfer to control its type and direction, as well as setting up the top 8 bits of the transfer address. |  |
|  |  | Bits | Description |
|  |  | 0-7 | A16-23 - Top eight bits of DMA transfer address. |
|  |  | 8 | BUSY - this read-only bit indicates that a DMA transfer has not yet completed, and so the DMA control registers should not be altered. |
|  |  | 9 | BW - byte/word - is this bit is set the next transfer will be byte wide (The least significant byte of DMD is always used). |
|  |  | 10 | RW - read/write - if RW is set to ONE the following DMA transaction(s) will be READ. |
|  |  | 11 | HOLD - when set to ONE, the DSP will assert HOLD to the host, until the HOLD bit is set to ZERO again. This bit should only be set under special circumstances - see below. |
| F18288 | 144 RW | DMA data. This register should be set up with DMA write data prior to initiating a DMA write, and will contain the DMA read data after a read. If a byte transfer is being performed, then only the bottom eight bits are relevant. The top eight bits are undefined after a byte read. |  |
| F1828A | 145 RW | Multiplier result 0. This is the bottom sixteen bits of the product after a multiply or multiply / accumulate. These registers may be written prior to a multiply / accumulate sequence to give an initial sum value. If the initial sum is zero, there is no need to write to them, just make the first operation in the sequence a multiply. |  |
| F1828C | 146 RW | Multiplier result 1. This is the top sixteen bits of the product after a multiply or multiply / accumulate. |  |
| F1828E | 147 | Multiplier overflow bits and ALU carry flag |  |
|  |  | Bits | Description |
|  |  | 0-4 | Z32-36 are the most significant five bits of the multiplier/accumulator result register. Only four of these bits are fed back into the accumulator. Z36 is provided merely as a means of detecting overflow in normal use. |
|  |  | 5 | CARRY is the ALU carry latch |
| F18290 | 148 R/W | Serial DAC Left. This is a sixteen bit signed integer, and is write only. Reading this register gives the left serial input value. |  |
| F18292 | 149 R/W | Serial DAC Right. This is a sixteen bit signed integer, and is write only. Reading this register gives the right serial input value |  |
| F18294 | 14A RW | Program counter. This register must be set up by the host before any DSP program is run. |  |
| F18296 | 14B RW | ALU Mode register |  |
|  |  | Bits | Description |
|  |  | 0-3 | S3-0 are the ALU mode bits for the GAI instruction, as described in the TI data book for the 74181, and in Appendix A. |
|  |  | 4 | M as above |
|  |  | 5 | TCX - if TCX is set to ONE then the X register will be treated as two's complement number for all MULT and MAC instructions. |
|  |  | 6 | TCYN - if TCYN is set to ZERO then the value (nn) will be treated as a two's complement for MULT and MAC instructions. Note that it has the inverse sense to TCX. |
|  |  | 7 | MAP1 - when set RAM and ROM are swapped in the data space |
| F18298 | 14C RW | ALU X register (first operand). |  |
| F1829A | 14D RW | ALU Z register (result). |  |
| F1829C | 14E | Intrude address register. See comments about intrude data register. |  |
| F1829E | 14F | DSP Control. Write only. |  |
|  |  | Bit | Description |
|  |  | 0 | Host Interrupt. If one is written to this location the DSP will generated an interrupt to the CPU. This is enabled through the video controller section. There is no need to write a zero to this location. |
|  |  | 1 | DSP Stop. The DSP can be stopped by writing a one to this bit. Writing zero has no effect. |
| F182A0 | 150 | Divide 0. Read - the least significant word of the quotient. This is the quotient for sixteen bit divides. Write - the least significant word of the divisor, or the divisor for sixteen bit divides. A write to this register initiates the divide. |  |
| F182A2 | 151 | Divide 1. Read - the most significant word of the quotient. Write - the most significant word of the divisor. This should be set to zero for sixteen bit divides. |  |
| F182A4 | 152 | Divide 2. Read - the least significant word of the remainder, but note that if this remainder value is negative then the divisor should be added to it to give the true remainder. This is the remainder for sixteen bit divides. Write - the least significant word of the dividend. This should be set to zero for sixteen bit divides. |  |
| F182A6 | 153 | Divide 3. Read - the most significant word of the remainder, see note above. Write - the most significant word of the dividend, or the dividend for sixteen bit divides. |  |
| F182A8 | 154 | Divide Control. |  |
|  |  | Bit | Description |
|  |  | 0 | DIV_OFFSET - When the divider is performing thirty-two bit divides, if this bit is set, the divider treats its operands at two sixteen point sixteen bit numbers, giving a sixteen point sixteen bit quotient. This bit is read/write. |
|  |  | 1 | DIV_32 - If this bit is set, the divider performs thirty-two bit divides, if it is clear, then sixteen bit dividers are performed. This bit is read/write. |
|  |  | 2-14 | unused |
|  |  | 15 | DIV_ACTIVE - if this bit is set, the divide unit is active and the quotient and remainder are not valid. This bit is read only. |
| F182B0 | 158 | DAC Control. Controls the DSP serial DAC interface. This is a synchronous serial input and output data stream. |  |
|  |  | Bit | Description |
|  |  | 0 | PASSTHRU - if this bit is set, the input and output serial data streams are connected directly, and the DSP has no effect. This bit is set by default after a reset. |
|  |  | 1 | INTERNAL - this enables the internal bit clock and word select generators, if this bit is clear then their timing is taken from the serial input. |
|  |  | 2-7 | RATE0-5 - sets the rate of the serial data clock. This is a programmable divide from the system clock, where the rate is given by clock divided by RATE+1. |
|  |  | 8 | CLK_POL - if this bit is set, the clock polarity is reversed from is usual sense, which is rising edge = valid data. |
|  |  | 9 | WRD_POL - if this bit is set, the word select polarity is reversed from its usual sense, which is high = right. Set this bit for Sony mode. |
|  |  | 10 | DAT_POL - setting this bit reverses the polarity of data for both input and output channels. |
|  |  | 11-12 | WORD_SIZE0-1 - these bits control the offset from a rising or falling edge of word select to the first bit of data, as follow: 0 1 clock Philips mode 1 8 clocks Sony mode 2 16 clocks Matsushita mode |
|  |  | 13-15 | unused |
| F182B20-2FE | 159-17F | Reserved for future expansion. |  |
| F18300-5FF | 180-37F | Map 0: 512 x 16 words of data RAM. |  |
|  |  | Map 1: 256 x 16 full wave two's complement SINE table |  |

### Program RAM

F18800-BFF	Host address of 512 x 16 RAM words
F18FF8		Extra bits register

DSP Program RAM is 512 18-bit words. These locations may only be written by the host processor, and are program read only as far as the DSP is concerned. Program RAM does not appear in the DSP internal address space. The RAM is not accessible to the host when the DSP is running.

The extra two bits, which correspond to the two most significant bits of the instruction field, are programmed by first writing their value to bits 0 and 1 of the extra bits register, and then writing a RAM location. If the extra bits register is just left set to zero, then 16-bit instructions can be written with a more restricted address range, and this provides compatibility with programs written for previous versions of the Slipstream chip.

### Run / Status Register

F18FF0	Host address

| Bit | Description |
| --- | --- |
| 0-2 | INTRUDE0-2 are the bits of the state machine controlling the INTRUDE mechanism. All three bits need to be zero in order for the host to initiate any INTRUDE transaction. |
| 4 | RUN/STOP - writing a one into this bit sets the DSP to RUN. Writing a zero to this port causes the DSP to stop. Reading this bit gives the RUN/STOP condition. |
| 5 | STEP - writing a one to this bit when the RUN bit is clear will single step the DSP. A zero is always read. |

WARNING - when single-stepping the DSP note that the instruction after a jump (MOV to the PC) is not executed. Single-stepping programs should either modify the PC to force this, or programmers should always follow a MOV to the PC with a NOP.

## DSP Instruction Set

Each instruction has a seven bit op code and an eleven bit address vector.

All micro-coded instructions (with the exception of multiply or multiply and accumulate) are completed in one 85ns cycle. All instructions are memory to register transfers or register to register transfers. Immediate values are not allowed. If a constant is needed in programming, then if it is not available in the constant table, a data RAM location must be set aside for the value.

µ §

This diagram shows how an instruction is made up. The instruction field denotes which instruction this is, the C field control s if the instruction is dependant on the carry flag, the X field enables the indexed addressing mode, the data address field gives the operand address. The host bit numbering shows how the instruction appears in the 16 bit program RAM space, with the X bits showing the bits programmed by the extra bits register. The instruction bit numbering shows the internal representation.

### Conditional Instructions

If bit 12 in the instruction code is SET, then the instruction is executed only if the CARRY bit in the ALU is also set.

For example, if the instruction is a memory move operation the register or memory location that would be updated will remain unchanged if the carry bit is not set. Similarly if the operation is an arithmetic type, then the result register will not be updated. This is true for all instructions except MULT and MAC. It is not possible to use condition codes with these last two instructions.

### Indexed Addressing

If bit 11 in the instruction code is SET, then the 9 bit address vector in the instruction code is added to the 9 bit value in the INDEX register IX, to produce the address in Data Memory operated on by the instruction.

### Op Codes

| No. | Mnemonic | Description |
| --- | --- | --- |
| 00 | MOV (nn), MZ0 | Move data from the least significant word of the multiplier result register into data location nn. |
| 01 | MOV (nn), MZ1 | Move data from the most significant word of the multiplier result register into data location nn. |
| 02 | MOV MZ0, (nn) | Move data word from data location (nn) to least significant word of multiplier result register. |
| 03 | MOV MZ1, (nn) | Move data word from data location nn to most significant word of multiplier result register. |
| 04 | CCF | Complement the carry flag |
| 05 | MOV DMA0, (nn) | Move data from (nn) to the least significant word of the DMA address register. Executing this instruction will start the DMA transaction in the manner described by the DMA1 register. The programmer must allow the minimum latency time to expire between two consecutive instructions of this type. |
| 06 | MOV DMA1, (nn) | Move data from data location (nn) to the most significant bits of the DMA address register. Control bits are also passed with this instruction. See note on DMA transfer |
| 07 | MOV DMD, (nn) | Move data from nn to DMA data register. |
| 08 | MOV (nn), DMD | Move data from the DMA data register to nn. |
| 09 | MAC (nn) | Multiply and Accumulate. Multiply the contents of nn with the contents of the X register and add the result to the number held in the multiplier result register [note 1]. |
| 0A | MOV MODE, (nn) | Move data from location nn to the ALU mode register. This register defines: a) The operation to be performed with the General Arithmetic Instruction. b) Selection of signed or unsigned multiply operations. c) Certain other DSP control data |
| 0B | MOV IX, (nn) | Move data from (nn) to the index register. |
| 0C | MOV (nn), PC | Move data from Program counter to nn. (useful for relative jumps, computed GOTOs etc.). |
| 0D | MOV X, (nn) | Move data from nn to ALU X register. |
| 0E | MOV (nn), X | Read data from ALU X register to data location (nn). |
| 0F | MULT (nn) | Multiply the X register to the data in (nn). |
| 10 | ADD (nn) | Add the data in location (nn) to the data in the ALU X register and place the result in the ALU Z (result) register. Ignore the carry flag but change it to reflect carry from the add. |
| 11 | SUB (nn) | Subtract the data in nn from the data in the X register. Ignore the carry flag but change it to reflect the condition NOT borrow. |
| 12 | AND (nn) | Perform logical AND between data location (nn) and ALU X register. |
| 13 | OR (nn) | Logical OR between the data in nn and the ALU X register. |
| 14 | ADC (nn) | Add the data in nn, the X register and the carry bit together. Carry flag is updated. |
| 15 | SBC (nn) | Subtract data in nn and the carry flag from the ALU X register. Carry flag is updated. |
| 16 | MOV (nn), AZ | Move the data from the ALU result register to nn. |
| 17 | MOV AZ,(nn) | Load the result register directly from location nn. |
| 18 | MOV (nn), Z2 | Load nn with the MAC overflow bits and the ALU carry bit. |
| 19 | MOV DAC1, (nn) | Data from nn is passed to the LEFT D to A converter. The value is signed for all DAC instructions. |
| 1A | MOV DAC2, (nn) | Data from nn is passed to the RIGHT D to A converter. |
| 1B | MOV DAC12, (nn) | Move data from nn to BOTH D to A converters. |
| 1C | GAI (nn) | General Purpose Arithmetic Instruction. Implements the operation defined by the code loaded into the MODE register. See Appendix. |
| 1D | MOV PC, (nn) | Jump instruction. Note that because instructions are pipe-lined the instruction immediately following the jump instruction in program RAM will be executed before the one at the location specified by the contents of nn. |
| 1E | NOP | Guess what. Conditional version is especially useful. |
| 1F | INTRUDE | At least one INTRUDE instruction must be put into the DSP program if any data transfer is to take place between host and DSP under host control. |

### Data Transfer between Host and DSP

### Under DSP control - DMA transfer

The DMA transfer mechanism is based upon the DSP becoming the bus master on the main system bus, accessing 'host' memory. The DSP is the highest priority bus master, and will therefore be granted the bus by the current bus master as soon as it is able to give it up. See the discussion above.

The worst case for giving up the bus is the 386SX processor, which can take a considerable time to release the bus.

DMA transfers are initiated by a write to the DMA0 register. Transfer status information and the high part of the address should already have been written to the DMA1 register, as should write data to the DMD register in the case of write transfers.

When a transfer is initiated the DSP requests the bus, and when it is granted to it, performs the transfer and releases the bus. Completion of this operation may be polled for, or the programmer may choose to allow the maximum possible latency to elapse before using read data and/or initiating another transfer. The maximum latency is of the order of ??? cycles (this figure to be confirmed). This mechanism is efficient in the use of the main memory bus as the DSP only actually owns the bus for the duration of the transfer.

A second bus acquisition technique may be performed which uses the HOLD bit in the DMA1 register to request the bus. This may be more efficient if the DSP wishes to perform multiple transfers consecutively, as the bus is not released between transfers. The HOLD bit in DMA1 has to be cleared before the DSP will release the bus. This mechanism is not generally recommended, as the DSP will generally end up owning the bus for significant periods without any activity, which is wasteful of overall memory bus bandwidth, and could potentially disturb CD DMA transfer.

If using the second technique, the DSP must first request the bus before performing any DMA transfer. It has no means of detecting that it has gained the bus, and must therefore wait the maximum possible bus latency time, which is ??? instructions. Once the DSP has acquired ownership of the bus it may then proceed to perform bus cycles. It may perform an arbitrary sequence of read and/or write cycles, and should give up the bus at the end of these. Each of these should be allowed a ??? instruction latency, or be polled for completion.

The direction of a DMA transfer is controlled by the DMA1 register, which may also used to acquire the bus by setting its HOLD bit. The RW bit determines the direction of the DMA transfer.

### Under Host Control - INTRUDE

All the internal memory is mapped into the Host address space. When the DSP is in STOP mode, the host may write program memory locations just as if they were normal host memory. When the DSP is running, however, the Program memory is not available to the host.

DSP data memory is only available by the INTRUDE mechanism. In order that DSP operations are not disturbed in any way, data transactions can only take place when the DSP is executing INTRUDE instructions. When the DSP is stopped, it may be considered to be effectively executing INTRUDE instructions constantly.

### Host to DSP transfer

1.	Ensure that the INTRUDE state machine is ready to accept new data. This is indicated by bits 0 - 2 in the status register being at zero. If the DSP program length or the maximum time between INTRUDE cycles is known, then it is legitimate for the host to wait this amount of time before being sure that the DSP is ready

2.	Write data to the memory location as normal

### DSP to Host transfer

1.	Ensure that the state machine is ready (as above).

2.	Read the memory location. The data read will be the data from the last INTRUDE location and not the desired data. The action of this read is to set up the address for the INTRUDE state machine to fetch the new data from.

3.	Wait for the state machine to become ready again, either by polling the intrude status or allowing a suitable delay. If intrude instructions are sufficiently closely spaced then no wait at all need be performed.

4.	Read the data.

### Intrude Status

The bottom three bits of the intrude status register indicate the current status of the intrude state machine. The values of interest are:

0	Idle

2	Awaiting INTRUDE instruction to perform read

4	Awaiting second host read of DSP to host transfer

6	Awaiting INTRUDE instruction to perform write

State 4 is of particular significance. This should be checked for if there is a possibility of a single read occurring, and a dummy read performed to clear it if necessary.

State 6 means that the interface has received a write from the 386 and an INTRUDE instruction has not yet been executed, therefore another write should not be performed.

### Using the Blitter

Host to DSP program RAM transfer may be performed using the blitter only while the DSP is not running. The blitter cannot access DSP data RAM.

### Illegal Instruction Combinations

### Multiply / Multiply-Accumulate operations

These instructions take two DSP machine cycles to compute and load the result in MZ0 to MZ2. The data input to this operation is NOT pipe-lined so while it is possible to follow a MAC or MULT instruction with an unrelated op code, it is important not to immediately follow a MAC or MULT with any of the multiplier related instructions.

All of the following code combinations will generate spurious results:

BAD1:		MULT	(nn1)
		MOV	X,(nn2)

This will corrupt the X register and change the result of the previous multiply

BAD2:		MULT	(nn1)
		MULT	(nn2)

Whilst the result of the second multiply will  be valid, it will corrupt the value of the first.

BAD3:		MAC  (nn2)
		MOV  (R_X), AZ

BAD4:		MAC (nn2)
		MOV  MZ0, (nn3)

### Serial DAC Interface

The serial DAC interface allows the DSP to both drive a synchronous serial (I2S or similar) DAC, and to input data from a synchronous serial data source such as a CD drive. The interface timing can be internally generated if no input device is attached, but if a data source is present, then it must be used to determine the timing.

An internal overflow detector prevents the DSP from writing to the DAC before the previous output data has been fully output, and this is governed by writes to DAC1. Therefore, DAC transfers should take the form:

·	write to DAC1 (or both)

·	write to DAC2 (if not both)

·	read input values

These should be performed in close succession (less than 16 instructions).

There is no detection of undeflow, and should this occur then the previous output value will be output again.

The DAC values are double buffered, so that although audio code should loop at an average rate less than or equal to the sample period, it is possible for occasional passese through the loop to take up to two sample periods. This may be useful for exception processing.

### The DSP Arithmetic Logic Unit

The DSP contains an arithmetic logic unit compatible with the Texas Instruments' 74181 device, and all its functionality is available by setting bits in the ALU mode register and using the GAI instruction. For those of you who don't have access to a TI data book here is a summary of the functions. Note that carry in to the ALU is given by the carry flag, and that carry sometimes means NOT borrow.

| S | M=1 | M=0 |  |
| --- | --- | --- | --- |
| 3210 |  | Cin=0 | Cin=1 |
| 0000 | /A | A | A plus 1 |
| 0001 | /(A+B) | A+B | (A+B) plus 1 |
| 0010 | /A.B | A+/B | (A+/B) plus 1 |
| 0011 | 0 | minus 1 | 0 |
| 0100 | /(A.B) | A plus (A./B) | A plus (A./B) plus 1 |
| 0101 | /B | (A+B) plus (A./B) | (A+B) plus (A./B) plus 1 |
| 0110 | A#B | A minus B minus 1 | A minus B |
| 0111 | A./B | (A./B) minus 1 | A./B |
| 1000 | /A+B | A plus (A.B) | A plus (A.B) plus 1 |
| 1001 | /(A#B) | A plus B | A plus B plus 1 |
| 1010 | B | (A+/B) plus (A.B) | (A+/B) plus (A.B) plus 1 |
| 1011 | A.B | (A.B) minus 1 | A.B |
| 1100 | 1 | A plus A | A plus A plus 1 |
| 1101 | A+/B | (A+B) plus A | (A+B) plus A plus 1 |
| 1110 | A+B | (A+/B) plus A | (A+/B) plus A plus 1 |
| 1111 | B | A minus 1 | A |

Notation

/	Boolean NOT (highest precedence)

+	Boolean OR

.	Boolean AND

#	Boolean exclusive OR

# Compact Disk Controller

The CD interface contains the following blocks:

·	simple synchronous serial interface, for I2S and similar

·	CD-ROM block decoder

·	DMA channel

The mechanism allows a serial data stream to be transferred to main memory, either directly, or by first passing through a block decoder. This allows an external block decoder to be used, in case of problems or incompatibilities in the internal one.

An interrupt can be generated when a transfer completes, given by the transfer length counter reaching zero.

## Synchronous Serial Interface

The following input data formats are supported.

This is the Philips data format. The word select leads the data by one tick, and the data is aligned against the MSB of a 32-bit datum. A low on word select indicates left data.

µ §

This is the Sony data format. The word select is co-incident with the MSB of a 24-bit datum. The sixteen bit data is aligned against the LSB. A high on word select indicates left data.

µ §

This is the Matsushita data format. The word select is co-incident with the MSB of a 32-bit datum. The sixteen bit data is aligned against the LSB. A low on word select indicates left data.

µ §

Also supported is the Philips block decoder output format. The bit ordering is reversed, and the first bit is aligned against the first bit of the datum. The word select format can be any of the above.

The interface has separate flags for each part of the various arrangements above, so that other combinations will be possible.

## Block Decoder

The block decoder synchronises to the start of the 2352 byte sectors, performs the de-scrambling, and computes the EDC to detect errors. It operates in either short mode, where 2048 data bytes are transferred after the header, or long mode, where the 2340 bytes after the sync pattern are transferred. This allows the header and error correction data to be read if desired. The header size is programmable to either 4 or 12 bytes to support CD-ROM mode 1 and CD-I / XA Mode 2 Form 1, but header matching is only performed on the main four byte header. The Mode 2 forms are only supported by operating in long mode and extracting the required data.

Header matching is performed on the first sector of a transfer to ensure that the correct data is being read. The desired header values should be programmed into the header registers. If a multi-sector transfer is performed then no further matching occurs after the first sector.

It is envisaged that normal transfers are performed in short mode, with the long mode being used when an error has been detected, so that the operating software can attempt to correct it.

Multiple sector transfers are supported by giving a count of the total number of long-words to be transferred. Errors will abort multiple sector transfers.

Errors can take the following forms:

·	Unreliable data. This is the logical OR of the error input (EFIN) over the block. This may not necessarily indicate an actual error.

·	EDC error. The EDC detected an error in the block, and corrective action should be taken. This should be ignored for Mode 2 blocks.

·	No sync. This is not detected by the hardware directly, but may be implemented by some form of time-out in the controlling software.

It is possible to poll the decoder to determine its current status.

The controlling state machine is:

µ §

This state machine advances every 32-bit word.

## DMA Interface

The DMA interface can transfer two 16 bit words at a time into host memory. It can take either the output from the internal block decoder or the output from the synchronous serial interface. It has an address counter that runs upwards through memory.

It has a transfer length counter for direct transfer from the serial interface.

In forever mode the DMA address regsiter wraps within a 32 Kbyte buffer, and the counter is ignored. This may be useful for CD audio data, or for real-time data handling, such as "full-motion video" de-compression. A CPU interrupt is generated every time the address pointer wraps around the buffer.

## Control Interface

The Host Control Interface has the following registers:

### CD_CMD	CD Command Register	60h	WO

| 0 | READLONG | Makes the block decoder DMA transfer all of the block except for the sync pattern, as opposed to just transferring the 2048 bytes of user data of Mode 1 or Mode 2 Form 1. This allows error correction to be performed, or alternate data formats to be read. |
| --- | --- | --- |
| 1 | SUBHEAD | Indicates that the block is Mode 2 Form 1, i.e. it contains 8 bytes of sub-header information between the header and the user data. |
| 2-3 | ERRMODE0-1 | Indicates the external decoder error flag position in the word: 0 Bits 6 and 14 (Philips format) 1 Bits 6 and 14 (Sony format) 2 Bits 8 and 24 (Matsushita format) Note that modes 0 and 1 are not the same as the numbering conventions are different. |
| 4 | POLARITY0 | Reverses the polarity of the serial clock input, normally positive edge triggered. |
| 5 | POLARITY1 | Reverses the polarity of the serial data input, normally high = 1. |
| 6 | POLARITY2 | Reverses the polarity of the serial error input, normally high = error. |
| 7 | POLARITY3 | Reverses the polarity of the word select input, normally low = left. This should be set for Sony format. |
| 8 | WSDEL | Delays the word select input by one bit, should be set for Philips format. |
| 9-10 | WSIZE0-1 | Indicates the number of clocks from the start of the word to the first valid data bit: 0 no delay (Philips format) 1 wait 8 bits (Sony format) 2 wait 16 bits (Matsushita format) |
| 11 | NOBLOCK | Disable the block decoder and DMA the serial input data directly. |
| 12 | MIRROR | Reverse the bit ordering within 16 bit words, normally first bit is MSB. |
| 13 | SHORTSEC | Test purposes only - write zero. |
| 14 | FOREVER | If this bit is set, the CD interface continuously writes data into a circular buffer 32K bytes long, and on a 32K boundary. The position of the first transfer is given by the initial value of the address register, and the transfer length counter is ignored. To stop operation, clear this bit, and write a small value into the counter. The CD interrupt will be generated every time the counter wraps. |
| 15 | NOMATCH | If this bit is set, no attempt will be made to match the block header. |

A write to the command register initiates CD operation.

### CD_ADDR0	CD Address Register 0	62h	WO
CD_ADDR1	CD Address Register 1	64h	WO

This pair of registers take a 24-bit address which must be long aligned. This is where DMA transfer will start. The bottom two bits of the value written are ignored.

### CD_CNT0	CD Transfer Length Counter 0	66h	RW
CD_CNT1	CD Transfer Length Counter 1	68h	RW

This 22-bit down counter is the number of long words of data to be DMA transferred. There is no requirement for it to be an integer number of blocks, exactly the number of long words specified will be transferred. When it reaches zero the block decoder stops operation at the end of the current block, unless FOREVER is set.

### CD_HDR0	CD Header 0	6Ah	WO
CD_HDR1	CD Header 1	6Ch	WO

This 32-bit register is the header of the first sector required.

### CD_STAT	CD Status Register	60h	RO

This gives the status of the CD interface.

| 0 | EDC_ERROR | Indicates that the computed Error Detection Code generated did not match the disk. The sector may be erroneous. If multiple sectors have been read, the error me be in any one of them. Cleared by a write to the command register. |
| --- | --- | --- |
| 1 | I2S_ERROR | This flag indicates that the I2S data input error flag was set at some point in the data that has just been read. The data may be unreliable. See the data sheet for the external decoder. This flag is cleared by a write to the command register. |
| 2 | CD_GO | Indicates that the CD interface and block decoder is active. Has no relevance if NOBLOCK is set. |
| 3 | EDC_BAD | Indicates that the last sector read had a bad EDC. Ignore this and use bit 0. |
| 4-15 | unused |  |